Measurement

This LO is massive, and covers essentially an entire textbook. For this reason we will return to it frequently.

BT_SQ 1.6  Describe the methods of measurement applicable to anaesthesia, including clinical utility, complications and sources of error in particular:

· SI units

· Measurement of volumes, flows, and pressures, including transducers.

· Measurement of blood pressure

· Measurement of cardiac output

· Measurement of temperature

· Oximetry

· Gas analysis, including capnography

· Methods used to measure respiratory function, including:

– Forced expiratory volume

–  Peak expiratory flow rate

–  Vital capacity

–  Flow-volume loops

–  Functional residual capacity and residual volume

28/11/16

| TRUE/FALSE A pulse oximeter radiating 2 wavelengths of light can only differentiate 2 different forms of Hb. | True |
| --- | --- |
| TRUE/FALSE Oxygenated haemoglobin absorbs light at a wavelength of 660nm. | True HbO2 - more at 940nm Hb - more at 660nm But they both also absorb light at each other's wavelengths (why you need to do a ratio of one to the other) |
| TRUE/FALSE The y axis on the plethysmograph is an estimate of arterial calibre and thus sympathetic tone. | True Lambert's law: the absorption of radiation as it passes through a substance increases exponentially as the distance it travels through the substance increases. Variability in the y-axis readings (independent of pulse waves) is due to increased thickness of the tissue due to arterial pressure increasing the calibre of the arteries. I think this is the maths behind using plethysmograph readings to estimate CO? |
| TRUE/FALSE Methaemoglobin is strongly absorbed at 660 and 940nm | True Methaemoglobin absorbs light very strongly at both 660nm and 940nm. Because the pulse oximeter measures the difference between the 2 wavelengths, it will affect both measurements, making the difference between then very small, even when there is a low concentration of metHb. When the ratio is the same, the pulse oximeter reads 85%. This is why methaemoglobin gives a reading tending towards 85% but not lower. |

| TRUE/FALSE An adult pulse oximeter cannot accurately read oxygen saturations when foetal haemoglobin is present. | False According to Aston equipment: not affected by the presence of different Hb types (HbF, HbA, HbS etc) |
| --- | --- |

29/11/16

Since you’ve already been reading about pulse oximeters we’ll stay on this topic.

| TRUE/FALSE Response time is faster when the oximeter is on the earlobe cf the finger | True |
| --- | --- |
| TRUE/FALSE Bilirubinaemia can result in a falsely low oxygen saturation with pulse oximetry | False |
| TRUE/FALSE The percentage of the signal which is pulsatile in finger pulse oximetry is approximately 80% | False About 2% - howequipmentworks.com |
| TRUE/FALSE Anaemia may cause under-reading of oxygen saturations with pulse oximetry | False Pulse oximetry doesn't differentiate the amount of Hb |
| TRUE/FALSE A pulse oximeter will detect a drop in oxygen tension from 600mmHg to 200mmHg | False Will only "detect" (estimate) PaO2 when it changes the SaO2 (Hb-O2 DC) |

30/11/16

Moving to measuring oxygen in its gaseous form…

| TRUE/FALSE An operating room paramagnetic analyser incorporates a pressure transducer | TRUE - alternating pressure at the transducer, the magnitude of which is a measure of the oxygen partial pressure in the sample gas. NOTE - old types had dumb bell |
| --- | --- |
| TRUE/FALSE Pressure exerted on the side of a tube decreases as flow rate increases | TRUE ? Bernoulli's principle? - Shrav's group weren't sure, I think it is true after reading this: http://physics.bu.edu/~duffy/py105/Bernoulli.html |

| TRUE/FALSE Nitric oxide at clinically used concentrations will falsely increase oxygen concentration in a paramagnetic analyser used in theatre | False Nitric oxide and O2 measured using PMGA - used in such low quantities 150ppm that doesn't affect the measurement - paramagnetic properties of NO are very small compared to O2 You could argue that it will increase the measured O2 conc but I think it would be so low that maybe it wouldn't even read (ie: less that 1%) |
| --- | --- |
| TRUE/FALSE Paramagnetic analysis degrades oxygen molecules into free radicals so the gas cannot be returned to the circuit | False Paramagnetic analysers just direct gases faster or slower down a tube, they do not change the molecules. Mass spectrometry "cracks" the molecules. |
| TRUE/FALSE Oxygen tension can also be measured with infrared analysis | False Infrared radiation is only absorbed by diatomic molecules (molecules that have two or more different elements in them): CO2, N2O, volatiles. It is not absorbed by molecules that only have one element in them: O2, N2, He |

"13/12/17" (belongs in a 20/12/16 but the blogger mucked up)

A contribution from an ex chair of the primary exam :

I was working in theatre with a registrar who had a previous degree in physics, and we were discussing the issues of accuracy with arterial monitors, in particular the concept of damping… Together we determined that this diagram probably illustrates some important concepts.

| TRUE/FALSE System A is an example of a system where the natural resonance frequency of the system is similar to that being measured. | FALSE It's underdamped. If the natural frequency of the system was equal to the fundamental frequency of the measured waveform, it would continue to oscilate indefinitely |
| --- | --- |
| TRUE/FALSE System B is the most ideal for a blood pressure measurement response because it provides the most accurate reading. | False Critically damped, (critically damped is the amount of damping where the signals returns to zero as fast as possible without overshooting) - would take too long to reach baseline each beat (may as well use a NIBP cuff) |
| TRUE/FALSE System C is an example of optimal damping as it provides a rapid response with minimal sacrificing of accuracy | True Optimal damping coefficient is 0.64 - compromise between overshooting and not taking too long to get back to zero, in physiological circuits this is 0.64 or 64% of critical damping. |
| TRUE/FALSE In all systems the eventual measured pressure will be accurate | True When damping coefficient =0 in a perfectly frictionless circuit, the measurement will never reach a baseline (will just continue to oscillate at the same amplitude). In reality (outside physicists' imaginations) this wouldn't be possible because there would be some resistance in the circuit to make this happen. Apparently however the MAP will be accurate. |
| TRUE/FALSE Damping is a reduction in the amplitude of an oscillation as a result of energy being drained from the system to overcome frictional or other resistive forces. | True |

15/12/16

Perhaps today’s topic, dynamic airway closure, will be of more interest as it is one that candidates really struggle with in vivas.

BT_SQ1.6 Describe the methods of measurement applicable to anaesthesia, including clinical utility, complications and sources of error in particular:
· Methods used to measure respiratory function, including:
– Forced expiratory volume
– Peak expiratory flow rate
– Vital capacity
– Flow-volume loops
– Functional residual capacity and residual volume

West describes the topic well in his book, where the following diagram is taken from. The red circle shows the net pressure gradient between the intrapleural space and the airway.

| Dynamic airways closure may occur during normal tidal breathing TRUE/FALSE | TRUE |
| --- | --- |
| Dynamic airway closure accounts for the effort dependent portion of the expiratory limb of the flow-volume loop TRUE/FALSE | FALSE Dynamic airway closure accounts for effort INDEPENDENT component Starlings resistor - Alveolar pressure - INTRPLEURAL pressure (not mouth) |
| During forced expiration, positive pressure generated will be transmitted equally across the respiratory system TRUE/FALSE | False Decreases along length of airway |
| The trachea is never subject to dynamic airway closure TRUE/FALSE | False Tracheal tug in babies Excessive dynamic airway collapse (EDAC) defines the pathological collapse and narrowing of the airway lumen by >50%, which is entirely due to the laxity of the posterior wall membrane : Pierdonato Bruno1 |
| During the effort independent part of an expiratory flow volume loop, maximum air flow rate is determined by lung volume TRUE/FALSE | True Maximal flow decreases with lung volume - FIG 7.16 WEST |

20/12/16

Once again I was with my registrar with the expert knowledge of physics waiting for the cardiothoracic registrar to take down the mammary.  It was as though time stood still.   We were both looking at the clock, and reminiscing on the wonderful mechanics of (non digital) clocks.  She commented to me in passing “I do miss the slow natural frequency of the pendulum of a grandfather clock” which made me consider the fast swinging pendulum of a cuckoo clock.   Needless to say, my mind turned to the concept of natural resonance frequencies in invasive pressure monitoring systems, and I thought back to the days of my music lessons…

| TRUE/FALSE The natural resonant frequency of a system is proportional to the stiffness or tension in the system, and inversely proportional to the mass. | TRUE Hence - decreased compliance and increased density (ie. Clots) - decr natural frequency of arterial set up Also Increased length decr nf |
| --- | --- |
| TRUE/FALSE As in tightening a violin or guitar string, increasing the stiffness or tension will lead to an increase in the natural resonant frequency (a higher note on the instrument) | True |
| TRUE/FALSE Like the pendulum of a grandfather clock being slower than that of a cuckoo clock on the wall, the pulmonary artery tracing on the monitor is not as good as the arterial system, as the pulmonary artery system has a much longer system and as such more mass and a lower natural frequency | ?TRUE • Has a lower natural frequency because LONGER tubing and hence increased mass • Therefore high frequency artefacts Above as per Shrav's group, I agree: https://books.google.com.au/books?id=45DKiUj1hLUC&pg=PA206&lpg=PA206&dq=natural+frequency+of+pulmonary+artery+trace&source=bl&ots=UOFpYsCxIv&sig=T68cxhHPIIA_uC7K_2n5DMURl2M&hl=en&sa=X&ved=0ahUKEwj0t9br57fUAhXHS7wKHZcRC_0Q6AEIKDAA#v=onepage&q=natural%20frequency%20of%20pulmonary%20artery%20trace&f=false |
| TRUE/FALSE The ideal system for an arterial monitoring system has a large length and very stiff tubing to ensure that its natural frequency is close to the frequency of the system being monitored | False You don't want the frequency of the system close to the natural frequency, otherwise you will get resonance pg 200 Aston |
| TRUE/FALSE The ideal frequency for a pressure monitoring system is determined by the pressure range being measured, rather than by the frequency of the system. | FALSE The frequency characteristics are independent of the pressures measured. The natural frequency (resonant frequency) of the measuring system should be at least 8 times the fundamental frequency of that being measured. 180bpm = 3 hz * 8 = 24hz |

22/12/16

BT_SQ 1.6 Describe the methods of measurement applicable to anaesthesia, including clinical utility, complications and sources of error in particular:
· Gas analysis, including capnography

| TRUE/FALSE The 90-95% response time for a CO2 analyser should be less than 150ms | ?True This article says 50-600ms is used clinically, but I would think 150ms would be better, especially high RR. |
| --- | --- |
| TRUE/FALSE Volatile agents can be distinguished from each other by measuring infrared absorbance at 3.3µm | False The absorption spectrum for volatiles is 8-13 μm range - although one graph I found showed desflurane with an absorption spectrum around 3, still I would be more likely to trust this source: https://www.ncbi.nlm.nih.gov/pmc/articles/PMC3821265/ |
| TRUE/FALSE Collision broadening means that the absorption peak for CO2 at 4.3µm is made wider in the presence of Nitrous Oxide | True |
| TRUE/FALSE Infrared analysers measure gas concentration rather than partial pressure | False Infrared analysers measure the partial pressure. - pressure usually displayed as continuous partial pressure vs time (capnograph) |
| TRUE/FALSE Water is a powerful absorber of infrared light | True Gas has to be dried before being analysed - passed through Nafion tubing (modified form of teflon) that is selectively and highly permeable to water - also in above book for Q1 |
| Follow on questions: 1. Why is the 90% response time important in a CO2 analyser? The answer is related to what you have learned about requirements for invasive pressure monitoring | 90% response time: the delay between sampling gas for CO2 measurement and the display of the measurement. Components of response time: - transit time - rise time Important because may affect: - changes capnograph shape - delay in capnograph compared reading (problem in critical intubations) - underestimation of ETCO2 (particularly in children due to rapid RR) - reduce the slope of phase II resulting in an underestimation of anatomical dead space.3,5 http://www.capnography.com/new/physics/chemical-method-of-co2-measurement?id=64 |

BT_GS 1.51 Describe the concept of depth of anaesthesia and how this may be monitored

10/1/17

| TRUE/FALSE The EEG of a patient when anaesthetised has a smaller amplitude than when they are awake | False Millers: |
| --- | --- |
| TRUE/FALSE Anaesthesia with ketamine produces a different pattern of brain EEG compared to anaesthesia with volatile agents | True |
| TRUE/FALSE If two patients have the same BIS number, then they are anaesthetised at the same depth of anaesthesia | ?False I feel like this is one of those all or nothing statements that just can't be true.. |
| TRUE/FALSE Administration of a muscle relaxant will reduce the BIS index if EMG is present | True |
| TRUE/FALSE The Entropy monitor measures the effect of anaesthetic drugs on the brain by calculating the randomness of the EEG | True |

BT_GS 1.55 Describe the concept of depth of neuromuscular blockade and explain the use of neuromuscular monitoring

2/2/17

| TRUE/FALSE Response to repeated single twitches at 1Hz is greater than at 0.1Hz | False At frequencies above 0.15Hz, the response will gradually decrease and settle at a lower level http://www.ld99.com/reference/ notes/text/Neuromuscular_monitoring.html Miller agrees |
| --- | --- |
| TRUE/FALSE In the late phase of recovery after muscle relaxant admnistration, tetanic stimulation can cause lasting antagonism of neuromuscular blockade | TRUE Reference as above |
| TRUE/FALSE The post tetanic count at which the first twitch on the train of four (TOF) appears is similar for both atracurium and cisatracurium | True - miller |
| TRUE/FALSE To completely prevent the bucking response to carinal stimulation the post tetanic count needs to be zero | TRUE - Miller fig 53.6 |
| TRUE/FALSE If no fade is felt with dual burst stimulation the train of four ratio is above 0.7 | ?True Double burst was developed because distinguishing face in TO4 was very difficult when ratio was >0.7 (supposedly would be able to distinguish), so if you can't determine fade in double burst then probably TO4 is >0.7 |

21/2/17

BT_SQ 1.6 Describe the methods of measurement applicable to anaesthesia, including clinical utility, complications and sources of error in particular

· SI units

· Measurement of volumes, flows, and pressures, including transducers.

· Measurement of blood pressure

· Measurement of cardiac output

· Measurement of temperature

· Oximetry

· Gas analysis, including capnography

· Methods used to measure respiratory function, including:

– Forced expiratory volume

–  Peak expiratory flow rate

–  Vital capacity

–  Flow-volume loops

–  Functional residual capacity and residual volume

| TRUE/FALSE If exhaled gas is not warmed to patient temperature in a pneumotachograph, volume will be underestimated | ?False Pneumotachographs make the flow laminar, then measure the differential pressure. - if the flow becomes turbulent: error in reading Viscosity varies with temperature - increased temp = increased viscosity - decreased temp = decreases viscosity Therefore: lower temperature = lower viscosity in the pneumotachograph if it is not heated. Reynold's number = vpd/viscosity - will be higher with lower viscosity - flow may become turbulent: this would just make the calculations inaccurate because the pneumotachograph is calculating the flow with the Hagan-Poiseuille equation rather than the turbulent flow equation. ALSO If the gas is colder, the viscosity will decreases, causing an increased laminar flow as per Hagan-Poiseuille. - this would cause it to OVERestimate. |
| --- | --- |
| TRUE/FALSE Volume is the area under a flow/time curve | TRUE. Flow = Volume/Time, therefore Volume = Time *Flow = AUC |
| TRUE/FALSE A pneumotachograph calculates flow from a known resistance and a measured pressure difference | True Pneumotachograph is a constant orifice (constant resistance), variable pressure flowmeter. |
| TRUE/FALSE A pneumotachograph uses the hydraulic version of Ohms Law | True |
| TRUE/FALSE A pneumotachograph measures flow accurately only when it is turbulent | False |
| This is a Dräger flow sensor from one of the limbs of a circle circuit. If you look closely you can see a fine wire between the top two prongs. The wire between the lower two prongs isn’t shown so clearly. What principle is this flow sensor using? | Thermal anemometry: hot wire. - is the most common method used to measure instantaneous fluid velocity. The technique depends on the convective heat loss to the surrounding fluid from an electrically heated sensing element or probe. If only the fluid velocity varies, then the heat loss can be interpreted as a measure of that variable. - the heat change relates to increased electrical resistance within the wire which can be measured (via wheatstone bridge) and then flow calculated. http://www-g.eng.cam.ac.uk/whittle/current-research/hph/hot-wire/hot-wire.html |
| Here are two different views of the sensor that modern GE machines use for measuring flow in a circle. The two tubes are hollow, and you can see on the photo on the left that they are open to the circuit. What principle is this flow sensor using? How do you think that it works? | A pneumotachometer: fixed orifice, differential pressure. - uses a restrictor in the gas flow passage to create a pressure drop that can be sensed by a differential pressure transducer. http://www.apsf.org/newsletters/html/2008/spring/08_dearsirs.htm |

26/5/17

Recently I was anaesthetising an adult with a congenital syndrome. I was quite worried about the airway—but in the end it wasn’t that which caught me out. She had no congenital heart disease, but had a pericardial effusion drained a few years previously. I was quite sparing with the induction agents as I wanted to maintain spontaneous respiration, but nonetheless…

About 5 minutes after induction, I noticed the blood pressure was 54/28.

BT_SQ 1.6

| T/F At low levels of blood pressure, the NIBP tends to give spuriously low values. | True |
| --- | --- |

| T/F The most accurate component of the NIBP is the mean. | True In oscillometric device: MAP is the largest amplitude of oscillations. |
| --- | --- |
| At the same time, her saturation dropped to 88, even though she was breathing 100% oxygen. The pleth had a good volume and looked normal. BT_SQ 1.6, BT_PO 1.29 T/F The fall in SpO2 was most likely to be artifactual. | True It is likely due to the low blood pressure. - plethysmography works off the difference in diameter between diastolic and systolic pulsations, the difference is already very low so if the pulsation is weak and causes an even smaller increase in diameter: pulse oximeter is inaccurate |
| I gave three doses of 1mg metaraminol, but, although the saturation improved, the blood pressure remained in the low 70s. Heart rate was in the 40s. Worried that I might see another fall in saturation I decided to run a noradrenaline infusion. BT_PO 1.52 In such a situation, the most appropriate vasoactive agent would be: a) Ephedrine b) Metaraminol c) Adrenaline d) Noradrenaline e) Isoprenaline | a This is what I would use, most readily available, longer duration of action as a bolus, has both alpha 2 and beta 1 agonist actions, safer peripherally, larger margin of error for titrating. Could use adrenaline infusion if you wanted to but don't have central access (although it can be run peripherally) and very small amounts would only give you beta 1, when you also need alpha 2 action. |
| After a 20µg bolus dose of noradrenaline, the heart rate dropped to 28. T/F The most likely cause of the fall in heart rate is alpha 1 receptor agonism in the SA node. I found out, after the (otherwise uneventful) operation, that she normally has quite a low blood pressure. A good reminder that, when having trouble with anaesthesia, one should first look to the proximal end of the needle. | False Reflex bradycardia |

These questions relate to ROTEM which seems to be the favourite viscoelastic assay in use at present. Even my humble little hospital has one of these things.

29/5/17

| Q. It takes about an hour to provide clinically useful information. TRUE/ FALSE | False The clot formation information should be ready earlier, fibrinolysis measurement may take about an hour. |
| --- | --- |

| Q. Will reliably detect platelet dysfunction as a result of clopidogrel therapy. TRUE/ FALSE | False Different additives to the ROTEM cup will be able to detect the effect of platelets on clotting - FIBTEM reagent contains ctyochalasin D (platelet inhibitor) so clot will be independent of platelet function - EXTEM reagent doesn't inhibit platelets: so difference between EXTEM and FIBTEM is the clotting action due to platelets However: you can't tell the difference between platelet dysfunction due to clopidogrel and that due to aspirin if patient is on both aspirin and clopidogrel, just overall platelet dysfunction TEG can tell the difference with difference reagents added - PlateletMapping system |
| --- | --- |
| Q. Reliably detects the presence of heparin therapy. TRUE/ FALSE | True Reagents: - HEPTEM: contains hepatinase - compared to INTEM - can detect coagulation inhibition due to heparin |
| Q. Can detect primary and delayed fibrinolysis. TRUE/ FALSE | True LY30 and LY60 - measures of clot strength at 30 and 60 minutes - gives indication of how quickly fibrinolysis is occuring |
| Q. Will be abnormal if the patient is hypothermic. TRUE/ FALSE | False You can alter the operating temperature of the machine so that it's the same as the patient - by comparing results of altered temp to 37C: can work out the effect temp is having on clotting - so would only be abnormal if you didn't know the patient was hypothermic |

Primary Daily LO

Analgesia

BT_GS 1.20 Outline the genetic variations in the cytochrome P450 2D6 enzyme and discuss the clinical relevance for drugs used in the perioperative period

This enzyme is responsible for much of the variation in efficacy and toxicity of some commonly used drugs.

| This enzyme metabolises tramadol into a more active metabolite TRUE/FALSE | TRUE M1 metabolite with high mu receptor affinity. |
| --- | --- |
| Ondansetron may be ineffective with poor metabolisers TRUE/FALSE | FALSE Metabolised in liver to inactives. P&H p291 |
| Patients from the middle east are more likely to be ultrarapid metabolisers TRUE/FALSE | TRUE More likely than what?! Sounds like population with greatest proportion of ultra-rapid metabolisers is Middle Eastern |
| Approximately 90% of caucasians are poor metabolisers TRUE/FALSE | FALSE Approx 10% Caucasians poor metabolisers |
| This enzyme metabolises codeine into a more active metabolite TRUE/FALSE | TRUE Demethylated to morphine. |

BT_PM 1.18 Describe the pharmacology of opioids deposited in the epidural space or cerebrospinal fluid

With regard to epidural fentanyl (1 mcg.kg-1):

| Plasma concentrations of fentanyl are similar to the same dose given intramuscularly TRUE/FALSE | TRUE ? Given high lipid solubility, plasma concentrations correlate well with CSF concentrations. |
| --- | --- |
| The duration of analgesia is 1 hour TRUE/FALSE | FALSE ? Can’t find anything specific but IT fentanyl at lower doses has a duration of effect ~1hr, so you would think epidurally administered fentanyl at higher doses would last longer. |
| The CSF concentrations of fentanyl are maximal at 50 minutes TRUE/FALSE | FALSE CSF concentration peaks in ~20mins, compared to 1-4hours for morphine. |
| The incidence of pruritis is greater than following an equi-analgesic dose of epidural morphine TRUE/FALSE | FALSE Morphine has greater chance of adverse effects. |
| The onset of analgesia is around 2-5 minutes because fentanyl is highly lipid soluble TRUE/FALSE | FALSE Yes highly lipid soluble but onset is still around 15-30mins. |

Sticking with neuraxial opioids:

| Intrathecal fentanyl has fewer adverse effects than intrathecal morphine TRUE/FALSE | TRUE Much higher lipid solubility of fentanyl limits it’s spread, therefore presumably fewer adverse effects. |
| --- | --- |
| Intrathecal morphine produces better postoperative analgesia than intrathecal fentanyl after LSCS TRUE/FALSE | TRUE Longer duration of action with morphine due to less lipid solubility. |
| Doses of intrathecal morphine greater than 50mcg have no greater benefits in terms of analgesia but carry a higher incidence of respiratory depression TRUE/FALSE | FALSE ? Usual dose of IT morphine is 100-300mcg. |
| 100 mcg of morphine is 0.1ml of morphine 10mg/ml TRUE/FALSE | FALSE! Maths is fun. |
| Extended-release epidural morphine is associated with less respiratory depression than IVPCA morphine TRUE/FALSE | FALSE ? Probably more resp depression with epidural morphine. This article seems to suggest no difference: http://www.frca.co.uk/Documents/230%20Neuraxial%20adjuvants.pdf |

BT_PM 1.15  Discuss the pharmacokinetic and clinical implications of different routes of administration for commonly used opioids, including the oral, transdermal, subcutaneous, intramuscular and intravenous routes, and with particular reference to fentanyl, morphine, methadone, tramadol and codeine

| Fentanyl undergoes significant first pass pulmonary uptake and metabolism. TRUE/FALSE | FALSE Significant first pass pulmonary uptake but don’t think there’s metabolism also. |
| --- | --- |
| The cytochrome P450 3A4 (CYP3A4) is predominantly responsible for the metabolism of Alfentanil. TRUE/FALSE | TRUE. Stoelting p236 |

| Alfentanil undergoes extensive hepatic metabolism that demonstrates extensive interindividual variability TRUE/FALSE | TRUE CYP3A4 metabolism - extensive interindividual variability. |
| --- | --- |
| The bioavailability of sublingual buprenorphine is similar to that of parenteral buprenorphine TRUE/FALSE | FALSE Sublingual buprenorphine bioavailability 50-65%. |
| Epidural fentanyl undergoes a biphasic absorption pattern TRUE/FALSE | ??? FALSE Can’t find anything to suggest it does!!! May says TRUE ? (From Niki) True When lipophilic opioids are injected into the epidural space as a bolus their systemic absorption pattern is biphasic. The initial 'portion' of the dose is absorbed relatively rapidly into the blood stream and quickly reaches the supraspinal centres, whereas the remaining 'portion' is initially distributed into the fatty tissues in the epidural space and is then absorbed into the blood stream more slowly, typically over the course of several hours [10]. This biphasic pattern sustains the blood-drug concentrations as compared with i.v. drug concentration... |

Anatomy

Q1 2016-A

| The vagus nerve is in the carotid sheath TRUE/FALSE | TRUE Carotid sheath contains Vagus, IJV and common carotid. |
| --- | --- |
| The glossopharyngeal nerve is in the carotid sheath TRUE/FALSE | FALSE |
| Pneumothorax is more likely with LIJ placement than RIJ TRUE/FALSE | TRUE ? Not too sure on this but sort of makes sense as RIJ easier - vein usually larger and straighter |
| The carotid pulse is lateral to the LIJ TRUE/FALSE | FALSE IJV is anterolateral to carotid. Unless very high up in the neck. |
| The LIJ has a greater calibre than the right TRUE/FALSE | FALSE |

BT_AM1.1 Describe the anatomy of the upper airway, larynx and trachea, including it’s innervation and endoscopic appearance.

| The turbinates project into the nasal cavity from the nasal septum. TRUE / FALSE | FALSE Nasal turbinates project to the nasal septum |
| --- | --- |
| When a curved laryngoscope blade is placed in the vallecula, it elevates the epiglottis by tensing the hyoepiglottic ligament. TRUE / FALSE | TRUE Laryngoscope blade in vallecula causes tension on hyoepiglottic ligament which pulls the inferior edge of the epiglottis anterior. |
| In an adult, cricoid pressure blocks the oesophagus by compressing it against the C3 vertebral body. TRUE / FALSE | FALSE Cricoid cartilage is at C6 level. |
| The sensory innervation of the area just above the vocal cords is from the external branch of the superior laryngeal nerve. TRUE / FALSE | FALSE. Sensory innervation by internal branch of SLN. External branch innervates cricothyroid muscle (increases vocal cord tension). |
| The surface landmark that corresponds with the carina is the manubriosternal joint (angle of Louis). TRUE / FALSE | TRUE Manubriosternal joint also corresponds with T4/5 disc. |

BT_PM 1.1 Describe the anatomy of the sensory pathways with particular reference to pain sensation

| The spinothalamic tracts are in the dorsal column TRUE/FALSE | FALSE The spinothalamic tracts are anterior and lateral. |
| --- | --- |
| Primary afferent neurons synapse in the dorsal root ganglion TRUE/FALSE | FALSE. Primary afferents synapse in the dorsal horn, 1-2 levels above where they enter the cord. |
| C-fibres synapse in the substantia gelatinosa TRUE/FALSE | TRUE Substantia gelatinosa is the area in dorsal horn where primary afferents synapse. |
| Pain and temperature fibres decussate at the level of the medulla TRUE/FALSE | FALSE Pain and temperature fibres are carried via the lateral spinothalamic tract. Decussation occurs 1-2 levels above where the primary afferents have entered the cord. |
| C-fibres are unmyelinated TRUE/FALSE | TRUE |

SS_PA 1.1 Describe the anatomy of the neonatal airway, how this changes with growth and development and the implications for airway management

| TRUE/FALSE Under extension of the neck may cause airway obstruction in the neonate | TRUE Prominent occiput leads to head and neck flexion at rest which may cause obstruction. Need head and neck in neutral position. |
| --- | --- |
| TRUE/FALSE Over extension of the neck may cause airway obstruction in the neonate | TRUE Over extension can also cause obstruction. |
| TRUE/FALSE Infants are obligate nasal breathers | TRUE. Until 5-6 months. |
| TRUE/FALSE Neonates have a large tongue in comparison to the oropharynx | TRUE |
| TRUE/FALSE Infants are unable to breathe via the mouth | FALSE ?? Can’t find anything specific, but I would think they are able, just much higher resistance via mouth breathing |

IT_AM 1.1 Describe the basic structural anatomy of the upper airway including the larynx

| TRUE/FALSE The cricoid is the only complete cartilaginous ring in the tracheobronchial tree | TRUE |
| --- | --- |
| TRUE/FALSE The vocal cord is formed by the superior edge of the cricothyroid membrane | FALSE Vocal cords formed by lateral portion of membrane (conus elastis). |
| TRUE/FALSE The recurrent laryngeal nerve supplies all the intrinsic muscles of the larynx | FALSE Cricothyroid muscle (increases tension on cords) is innervated by external branch of superior laryngeal nerve. |
| TRUE/FALSE The superior laryngeal nerve is purely sensory | FALSE Motor supply to cricothyroid. |
| TRUE/FALSE The posterior cricoarytenoids are the only muscles that open the glottis | TRUE Only muscles that abduct the cords. Adduction of cords by transverse arytenoid and lateral cricoarytenoid muscle. |

Cardiovascular

Draw both aortic root and a radial artery pressure wave forms on the same axes.  Explain the differences between them.

| Tapering contributes to the higher peak pressure seen in the radial artery TRUE/FALSE | FALSE ? Can’t find anything referring to tapering. Higher systolic in radial due to reflected waves. |
| --- | --- |
| Reflection contributes to the diastolic hump seen in the radial artery TRUE/FALSE | TRUE Diastolic hump is the upper part of dicrotic notch, caused by reflection and resonance. |
| Stiff vessels will transmit reflected pressure waves faster TRUE/FALSE | TRUE Higher pressure due to reduced compliance should mean faster flow and therefore faster transmitted pressure waves. |
| Mean pressure is higher in the radial artery TRUE/FALSE | FALSE Slightly lower MAP more peripherally. |
| The Bernoulli effect created by a slower flow rate when the aortic valve closes creates the incisura TRUE/FALSE | FALSE Incisura coincides with AV closure, caused by transient reverse flow (aortic pressure > LV pressure) before AV closes. Can’t find any reference to Bernoulli effect. |

BT_PO 1.49 Describe the cardiovascular changes that occur with ageing

| Ageing causes a decreased number of sinus node cells in the heart, leading to decreased B adrenergic sensitivity T/F | FALSE Sinus node cells do reduce with aging, however reduced B adrenergic sensitivity is due to B adrenoceptor downregulation. |
| --- | --- |
| The velocity of the transmitted pulse wave in the vasculature tends to be slower in the eldery T/F | FALSE Faster due to less compliant vessels. |
| Elderly have higher levels of baseline sympathetic nervous system activity T/F | TRUE Baseline activity is elevated but reduced reactivity. |
| The atrial component of ventricular filling is especially important in the elderly T/F | TRUE Contributes up to 30% to ventricular filling compared with 10% in young people. |
| Older patients often have an increased arterial pulse pressure due to reduced vascular compliance T/F | TRUE Less vascular compliance enhances the reflection of pulse waves, augmenting systolic pressure. Diastolic pressure may be reduced. Therefore higher PP. |

# SS_PA 1.21 Describe the foetal circulation

| Within the umbilical cord, there are two veins, and one artery. TRUE / FALSE | FALSE 2 umbilical arteries carrying blood to placenta, 1 umbilical vein carrying blood away |
| --- | --- |
| Foetal blood returning from the placenta has an oxygen saturation of 80%. TRUE / FALSE | TRUE Umbilical vein O2 Sat usually ~80%. Umbilical artery O2 Sat usually ~55-60%. |
| The foramen ovale remains open in the foetus under the influence of prostaglandins. TRUE / FALSE | FALSE It’s the ductus arteriosis that is kept open by the vasodilating effects of PGE2 produced by placenta. Foramen ovale kept open by pressure. |
| The ductus arteriosus adds blood into the aorta distal to the vessels supplying the brain. TRUE / FALSE | TRUE Insertion is distal to brachiocephalic trunk, left common carotid and left subclavian. |
| Following delivery, there is a “transitional” circulation. Target SpO2 levels for a neonate are: 70-90% at 3 mins, and 85-90% at 10 mins. TRUE / FALSE | TRUE Targets as per resus guidelines. |

BT_PO 1.48 Discuss the cardiovascular responses to:

· Changes in posture

· Exercise

· Valsalva maneouvre

· Positive pressure ventilation and PEEP

· Pneumoperitoneum

· Haemorrhage and hypovolaemia

· Surgery and trauma

| The blood volume impacts on the cardiovascular responses of a patient undergoing the Valsalva Manoeuvre TRUE/FALSE | TRUE Greater blood volume means less hypotension in phase 2. |
| --- | --- |
| Pneumoperitoneum with an intraabdominal pressure of greater than 10mmHg is likely to result in an increase in cardiac output TRUE/FALSE | TRUE Low IAP increases venous return via compression of the splanchnic circulation. IAP greater than 10mmHg compresses IVC, reducing VR and thus CO. |
| Compensation which occurs following haemorrhage, aims to restore arterial blood pressure to normal TRUE/FALSE | TRUE |
| Sympathetic stimulation associated with major haemorrhage results in significant coronary artery constriction TRUE/FALSE | FALSE |
| The baroreceptor setpoint changes during excersice TRUE/FALSE | TRUE |
| Coronary blood flow may increase 4 times resting level during heavy exercise | ?TRUE Most sources say may increase 5 times. |
| The cardiac output during exercise is higher than that calculated on heart rate and increased contractility alone | TRUE Increased SV also contributed to by increased LVEDV. |
| 1000 ml of interstitial fluid can be absorbed during shock during the first hour | TRUE Reduced capillary hydrostatic pressure may mean Net Filtration Pressure favours reabsorption into intravascular space. |
| The Valsalva Manoeuvre can be used to test the integrity of the baroreceptors | TRUE |
| A cerebrally mediated activation of the sympathetic nervous system occurs when the arterial pressure falls below 50 mmHg | TRUE CNS ischaemic reflex - results in massive sypathetic outflow and adrenaline release from adrenal medulla. Unclear exact mechanism. |

BT_PO 1.53 describe the pharmacology of adrenergic agonists

Here is one true statement to get you oriented:

Phenylethylamine can be considered the parent drug on which all sympathomimetics are based.

Here is the molecule with the carbons numbered

| Maximal ⍺ and B potency is conferred by OH substitutions on the 3′ and 4′ positions of the benzene ring T/F | TRUE ?? beta carbon OH substitution. (From Niki) False Lack of OH on 3 and 4 positions decreases affinity for adrenoceptors, not potency. |
| --- | --- |
| Sympathomimetics can be chiral around either the ⍺ or B carbon T/F | TRUE Eg: ephidrine and metaraminol are chiral at alpha and beta carbon |
| A very large substitution on the terminal amine promotes B1 selectivity T/F | TRUE Larger substitutions seem to confer B1>B2 selectivity. Eg isoprenaline is beta selective but not more B1 or B2. Dobutamine has a massive substitution on terminal amine and is more B1 selective. |
| A methyl substitution at the ⍺ carbon prevents metabolism by monoamine oxidase and prolongs duration of action T/F | TRUE MAO needs unsubstituted alpha carbon with maximum of CH3 on terminal amine. |
| Absence of hydroxyl groups on the benzene ring improves oral bioavailability by preventing metabolism by COMT T/F | FALSE Absence of hydroxyl groups improves oral BA due to being more lipid soluble rather than preventing COMT metabolism (which is also true) |

Describe the pharmacology of drugs used to manage acute or chronic cardiac failure, including: sympathomimetics, phosphodiesterase inhibitors, digoxin, diuretics, ACE inhibitors, nitrates and beta blockers

| TRUE/FALSE Digoxin is highly protein bound, hence the introduction of another highly protein bound drug is likely to precipitate toxicity | FALSE Digoxin only ~25% PPB. Plasma levels are increased by amiodarone, capropril and erythromycin. |
| --- | --- |
| TRUE/FALSE Digoxin is primarily excreted renally, and frequently patients who are prescribed digoxin may have borderline or impaired renal function which may precipitate toxicity | TRUE. |
| TRUE/FALSE Overdosage of digoxin may be treated using Digibind (R) which is an example of an immunoglobulin which binds to digoxin to cease its therapeutic effect | TRUE |
| TRUE/FALSE Hypokalaemia is frequently seen in digoxin toxicity, reflecting digoxin’s inhibition of the Na/Ca pump | FALSE More likely to see hyperkalaemia. |
| TRUE/FALSE The bioavailability of digoxin is reasonably high (approx 70%) allowing for oral loading doses to be only slightly slower in reaching therapeutic effects compared with intravenous loading doses | TRUE |

Drug Variability

SS_OB 1.9 Describe the influence of pregnancy on the pharmacokinetics and pharmacodynamics of drugs commonly used in anaesthesia and analgesia

| TRUE/FALSE In pregnancy the average gain of 8 litres of total body water significantly increases the volume of distribution of hydrophilic drugs. | TRUE |
| --- | --- |
| TRUE/FALSE Foetal and placental tissues provide another compartment for drug distribution. | TRUE |
| TRUE/FALSE Pseudocholinesterase activity is decreased in pregnancy causing prolongation of succinylcholine block. | TRUE Reduced by up to 30% from 10th week of gestation up to 6 weeks post partum. |
| TRUE/FALSE Pregnancy reduces MAC by 25-30%. | TRUE Some sources say 40%. |
| TRUE/FALSE Nociceptive response thresholds are elevated in pregnancy. | TRUE Can’t find much on it but makes sense. |

Monday Topics:

SS_OB 1.1  Describe the physiological changes and their implications for anaesthesia that occur during pregnancy, labour and delivery, in particular the respiratory, cardiovascular, haematological and gastrointestinal changes.

| Normal physiological changes begin in the first trimester of pregnancy. TRUE/FALSE | TRUE |
| --- | --- |
| The largest increase in cardiac output in a pregnant woman occurs immediately after delivery. TRUE/FALSE | TRUE Due to autotransfusion from final uterine contraction, reduced vascular capacitance from loss of placenta and release of aortocaval compression. |
| The closing capacity in normal pregnancy does not change. TRUE/FALSE | TRUE FRC (ERV+RV) is reduced and may approach CC. Other lung volumes remain much the same. |
| Gastrin is secreted by the placenta TRUE/FALSE | TRUE Therefore have higher than normal gastric acid secretion. Combined with reduced LOS tone means more reflux. |
| Progesterone from the gestational sac may cause changes in the renin-angiotensin-aldosterone system in the first trimester, promoting sodium absorption and water retention. TRUE/FALSE | FALSE RAAS upregulation due to oestrogen |

Equipment and Flow

# BT_SQ 1.12 Describe the principles and safe operation of vaporizers

ANAESTHETIC MACHINE

| A fresh gas outlet must have a standard 22mm, 15mm or 8.5mm connector compliant with ISO 5356. | T ISO Website (International Organization for Standardization) 8,5 mm sizes intended for use in paediatric breathing systems; 15 mm and 22 mm sizes intended for general use in breathing systems; 22 mm latching connectors (including performance requirements); 23 mm size intended for use with vaporizers, but not for use in breathing systems; 30 mm size intended for the connection of a breathing system to an anaesthetic gas scavenging system. |
| --- | --- |
| An anaesthetic machine must contain an integral high pressure relief valve to prevent high pressures in the breathing system | F – Must be present within the breathing circuit – but is not an integral part of the anaesthetic Machine – AS per college document PS54 – Anaesthetic Machine |
| The scavenging connector to the circuit must be a 22mm tapered conical fitting compliant with ISO 5356 | F – As per answer to question 1 - Should be 30 mm |
| An anaesthetic machine which requires electrical power must have a backup power supply which permits normal operation for at least 20 mins | F – Must have a back up power supply that permits normal operation for 30 minutes |
| The emergency oxygen flush must have a locking feature to facilitate use in airway emergencies | F – Can’t find source – but I doubt you can lock, due to risk of barotrauma I have read they are placed in a recessed setting and will deactivate as soon as the finger activating the switch is removed |

SUCTION

BT_SQ 1.11 Describe how medical suction is generated and how to set up and test suction systems, both fixed and portable

| Each piped suction outlet free airflow must be at least 40 litres/minute in an operating theatre | F – Should be 160l/min in OT, in anaesthetic room 40l/min |
| --- | --- |
| Negative pressure is generated by an electric motor and pneumatic pump using the Bernoulli’s principle | F – Central pump generating a negative pressure |
| The central Vacuum pump must be able to generate negative pressure of 60 kPa | T |
| The suction tubing should be connected directly to the wall outlet | F – Tubing can be connected to disposable bottle |
| Portable suction using an oxygen cylinder makes use of the Venturi principle | T |

Source Used: http://sydney.edu.au/medicine/anaesthesia/resources/lectures/suction_clt.html

VAPORISERS

BT_SQ 1.12 Describe the principles and safe operation of vaporisers

| The Aladin cassette vaporiser is an example of an injection vaporiser system | F – It’s a plenum bypass vaporizer, individual casettes containing the volatile are added to effectively a vaporizing chamber |
| --- | --- |
| Modern vaporisers use an electrical heating coil to compensate for the cooling caused by latent heat of vaporisation | F – Thermal conductivity, through the use of an outside copper jacket to conduct ambient temperature to vaporizer is used |
| A plenum vaporiser is designed so that the gas leaving the bypass is fully saturated under normal conditions | F – Gas leaving the vaporizing chamber is fully saturated |
| TRUE/FALSE Thymol in sevoflurane can cause the bimetalic strip of older vaporisers to stick | False, it is the thymol in halothane that causes the bimetallic strip to stick |
| The Quick-Fill system is used only for sevoflurane | ?F – couldn’t find, but you can quick fill desflurane so I assume this is false |

All referenced except from last one from

http://sydney.edu.au/medicine/anaesthesia/resources/lectures/gas_supplies_clt/vapourisers.html

BT_SQ 1.12 Describe the principles and safe operation of vaporisers

| They are heavy because they are full of liquid | F – Heavy because of thermal conductive metal jacket |
| --- | --- |
| Must be positioned in a particular order if there are two different vaporizers on the machine | T – Courtesy of cat obsessed Dr Muir, Position from Lowest SVP/Potency closed to circuit to highest - Not an issue if interlocking exists which is mandatory in Australia - Vapor can contaminate the vapor in vaporizer in front - Better explanation in sourve below |
| All require power to operate correctly | F – Tec 6 and aladin do, but not the Tec 5 |
| A Desflurane Tec 6 vaporizer does not require adjustment when used at altitude | F – Does require adjustment Partial Pressure of Volatile has clinical Effect Refer to source below for better explanation |
| Should not be laid on their side | ?F – I guess with modern vaporizers they can be left on their side due to valves etc |

OXYGEN DELIVERY SYSTEM

Compare and contrast oxygen delivery via nasal cannulae (nasal prongs/specs), simple face mask (eg Hudson or CIG mask) and Venturi mask.

| Having an end-expiratory pause will affect the performance of the Hudson mask | T – Pause will allow for an O2 reservoir to develop within pask |
| --- | --- |
| Having a high peak inspiratory flow will affect the performance of nasal cannulae | True – Variable performance device – inspiratory flow greater than that delivered will entrain air from the atmosphere – reducing FiO2 |
| Low flow rates may lead to an increased inspired pCO2 with the Hudson mask | True |
| The Bernoulli effect is relevant for Venturi mask function | True |
| With nasal cannulae the nasopharynx acts as an oxygen reservoir | True – Aston Equipment Book |

OXYGEN DELIVERY SYSTEM

BT_SQ 1.14 Describe different systems to deliver supplemental oxygen and the advantages and disadvantages of these systems

| A non rebreather mask has a one way valve which prevents entrainment of atmospheric air | F, also known as a reservoir mask – One way valve prevents expiratory gas from going into reservoir bag Aston |
| --- | --- |
| Venturi oxygen delivery devices make use of the Bernoulli principle | T |
| Delivering oxygen at 6L/min via a Hudson mask, a patient with a respiratory rate of 6 breaths per minute (bpm) will recieve a higher FiO2 than a patient with a respiratory rate of 12 bpm | T, Higher respiratory rate associated with greater inspiratory flow rates – thus entrainment of air will occur |
| Nasal cannulae are an example of a variable oxygen delivery device | T – At flows greater than nasal cannulae delivery – air will be entrained |
| 100% oxygen can be delivered to a patient using a Venturi mask with a rating of 1.0 | F – a fixed FiO2 can be delivered – however not an FiO2 of 1.0 – air entrainment via variable side ports |

NERVE STIMULATORS

BT_RA 1.9 Describe the principles of nerve stimulation to locate nerves and the safe use of nerve stimulators

| A short stimulation pulse is an advantage in a nerve stimulator for nerve localisation for neural blockade | T, short impulses will preferentially stimulate larger nerve – longer impulse are more likely to cause pain or stimulate adjacent muscle |
| --- | --- |
| A nerve stimulator for nerve localisation should have a constant voltage output | T, I think – it says a nerve stimulator has a constant current generator and frequency and duration is altered by an oscillator – which interrupt the constant current |
| When performing a nerve block with a nerve stimulator, the needle should be connected to the positive electrode | T – needle is the cathode (Think this is wrong, the negative terminal should be connected to the electrode closest to the target nerve or the stimulator needle - Aston and NYSORA, the cathode is negative and the anode is positive? - Niki) |
| When performing a nerve block with a nerve stimulator, injection of 5% dextrose increases current density at the tip of the needle | T – 5% dextrose is a non-conductive solution, thus the current at the tip of the area is present over a small area – as opposed to LA solution, which is a conductive solution, which will spread the area into which the current disperses – reducing current density |
| Nerve stimulators can be used to locate nerves with no motor innervation | F – Can’t find anywhere, but just thinking how else would you determine where a nerve is without stimulating the muscle it contracts (Think this might be wrong too, you can cause pain if you use a longer impulse, so I guess this would pick up pain afferents) |

Source for first 3 is aston

Source for the 4th is http://journals.lww.com/anesthesia-analgesia/fulltext/2005/06000/The_Electrophysiological_Effect_of_Dextrose_5__in.51.aspx

EEG

# BT_GS 1.52 Explain the principles involved in the electronic monitoring of depth of sedation, including EEG analysis.

| The EEG during sevoflurane anaesthesia has less “randomness” than when awake. | T |
| --- | --- |
| Propofol causes burst-suppression of the EEG at levels which have little effect on spinal reflexes. | Not sure cannot find anywhere – high doses |
| When burst-suppression is induced by propofol, total brain oxygen consumption is reduced by up to 90%. | Not sure cannot find anywere, 90% seems like a significant reduction in O2 consumption though – so maybe false Niki: False: 60% Barbiturates, etomidate, and propofol decrease the CMR and can produce burst suppression of the electroencephalogram. At that level, the CMR is reduced by approximately 60%. Because blood flow and metabolism coupling are preserved, CBF is decreased. - Millers |
| Nitrous oxide causes similar changes to the EEG compared to sevoflurane. | F – Nitrous oxide alone, initially causes a reduction in amplitude and frequenc, but with analgesia and depressed consciousness – fast oscillatory activity is seen |
| Electrocortical silence cannot be produced with ketamine | T |

FLOW

Describe the methods of measurement applicable to anaesthesia, including clinical utility, complications and sources of error in particular

| If exhaled gas is not warmed to patient temperature in a pneumotachograph, volume will be underestimated | Not sure – It is affected by temperature – my only thought was a reduction in temperature causes a reduction in viscosity, which according to hagan poiseulle will cause an overestimation of flow and thus volume – so I guess my answer would be False |
| --- | --- |
| Volume is the area under a flow/time curve | T – Volume is equal to flow x time |
| A pneumotachograph calculates flow from a known resistance and a measured pressure difference | T – Fixed orifice variable pressure |
| A pneumotachograph uses the hydraulic version of Ohms Law | T |
| A pneumotachograph measures flow accurately only when it is turbulent | F - Pneumotachograph measures laminar flow |

IV INDUCTION AGENTS

# BT_GS 1.59 Describe the pharmacological principles and sources of error with TCI

| A TCI using the Marsh algorithm will give the same dose of propofol to an eighty year old and a twenty year old patient of the same weight | T – Although you plug weight into the marsh model, this is only done so that the pump can tell you that this model is not devised for pts under 16 Weight is used to determine volume compartments, rate constants are fixed with the marsh model |
| --- | --- |
| TCI can be used for morbidly obese patients | F – Inaccurate – as lean body mass determined using Schneider models is based on an algorithm, where extremes of weight creates a negative LBM Schneider is capped at a BMI of 43, and marsh is capped at a weight of 150 kg |
| The Minto algorithm for Remifentanil was devised by an Australian anaesthetist | T |
| Plasma or effect site TCI can be used effectively for the Schnider algorithm | F – Plasma site is best for sedation not TIVA – very low induction doses are given |
| The James equations are used to calculate LBM in the Minto and Schnider algorithms | T |

Remifentanil + Propofol

Propofol and remifentanil target controlled infusions are often given together as a total intravenous anaesthesia technique. Discuss pharmacological reasons why this is a useful combination.

| There are significant pharmacokinetic interactions between these drugs | F |
| --- | --- |
| There are significant pharmacodynamic interactions between these drugs | T – Synergism between the two |
| Both drugs have a rapid offset | T |
| Adding remifentanil to propofol can lead to more stable haemodynamics | T |
| Can be used in patients susceptible to malignant hyperthermia | T |

TCI

BT_GS 1.59 Describe the pharmacological principles of and sources of error with target controlled infusion

| Inaccurate drug delivery from the infusion pump contributes to 55% of the overall inaccuracy of a TCI infusion | Could not find figures |
| --- | --- |
| With most modern TCI algorithms actual plasma concentrations are within 20-30% of predicted concentrations 95% of the time | Could not find figures Niki: The book recommended by website: - The correlation of predicted and actual plasma concentrations of propofol by the pump is quite poor but again probably not clinically important as you will be operating well above the Cp50 for amnesia. The error is about 20% and gets worse the longer an infusion runs. (Rathie, the first year pg 172) |
| The Marsh model uses age and weight to calculate the compartment size | F – Uses weight only |
| The Schnider model may calculate a negative lean body mass in very obese patients | True – based on james equation |
| The most clinically reliable method is to target the effect site concentration observed at loss of consciousness. | I guess true – going to be more accurate than the Bristol technique |

INHALATIONAL AGENTS

| An Entonox cylinder contains a mixture of liquid and vapour | F – Contains gas only, as it is stored above its pseudo-critical temperature of -6C |
| --- | --- |
| An Entonox cylinder is coloured French blue and white | T |
| The Poynting effect refers to how interactions between two different gases leads to a change in their physical properties | T |
| Entonox provides about 0.5 MAC. | T – roughly 50% Nitrous |
| Can cause megaloblastic anaemia | T – Effects of Nitrous |

| Nitrous oxide is more soluble in blood than desflurane | T – blood gas of nitrous is 0.47, whereas des is 0.42 |
| --- | --- |
| Nitrous oxide is more soluble in blood than nitrogen | T |
| Nitrous oxide undergoes approximately 10% hepatic metabolism | F |
| Nitrous oxide increases plasma homocysteine levels | T – thus its previous implication with CV risk |
| Nitrous oxide is an analgesic agent | T |

| A partition coefficient of 1.4 means that at equilibrium the partial pressure in blood is 1.4 times that in the alveolus | F – Partial pressures are equal, amount of gas differs |
| --- | --- |
| Saturated vapour pressure increases with temperature | T |
| Boiling point decreases with falling atmospheric pressure | T – Boiling point is when SVP is equal to atmospheric pressure – atmospheric pressure decreases with altitude |
| Maximum sevoflurane concentration at sea level is 33% | F – Determined by SVP/Atmospheric pressure – roughly 20% |
| Using desflurane in a sevoflurane vaporiser would result in a sub anaesthetic desflurane dose | F – Increased concentration, with potential boiling at room temperature occurring |
| CYP3A4 is responsible for the oxidative metabolism of halogenated inhaled anaesthetic agents | FALSE Cytochrome P450 2E1 |
| Desflurane is less likely to cause hepatitis than Isoflurane | TRUE Desflurane has less metabolism by 2E1 Halothane 20%, Sevoflurane 2% and Enflurane, 0.2 Isoflurane and 0.02 Desflurane Rare cases of isoflurane hepatotoxicity similar to that of halothane due to immunogenic reactive intermediates |
| Intrarenal metabolism of methoxyflurane can lead to oliguric renal failure | FALSE High output renal failure with fixed specific gravity/poorly concentrated urine. Vasopressin resistant. |
| Compound A exposure during anaesthesia causes a transient rise in blood creatinine and urea in humans | FALSE Compound A is a dose-dependent nephrotoxin in rats however threshold in Humans is controversial. Some studies demonstrate tranisent changes, however bulk of studies do not. However, FDA recommends fresh FGF of 1L/min for up to 1 hour and 2L/min for >1hr to avoid clinically significant levels of compound A |
| Desflurane and Sevoflurane may produce significant quantities of carbon monoxide when used with dry CO2 absorbents | TRUE CO2 absorbers contain strong bases that can extract labile protons from anaesthetic molecules resulting in CO. Soda Lime and Baralime can result in >30% CO. Des > enflurane > isoflurane > halothane > sevo And Baralyme > Soda lime |

| TRUE/FALSE Nitrous oxide produces analgaesia through actions on opioidergic neurons in the periaqueductal gray matter | TRUE N2O analgesic effect abolished by naloxone. Thought to stimulate encephalin which bind to opioid receptors that trigger descending noradrenergic pathways |
| --- | --- |
| TRUE/FALSE Inhaled anaesthetic agents probably have their action through inhibition of axonal propagation | TRUE (? Not completely explained) Molecular mechanisms still poorly understood. Evidence supports effects on membrane proteins including ligand and voltage gated ion channels of excitable cells |
| TRUE/FALSE Halogenated inhaled agents increase the affinity of the GABA receptor for GABA | TRUE Halogenated inhaled agents potentiate GABA A receptors and two-pore domain K channels. Whereas N2O and Xe inhibit NMDA channels |
| TRUE/FALSE Inhaled anaesthetics most likely have their action by perturbing the biophysical properties of lipid bilayers | TRUE (? Not completely explained) According to the modern lipid hypothesis, anesthetics do not act directly on membrane protein targets but rather perturb specialized lipid matrices at the protein lipid interface which act as mediators. This then affects ion channel function. |
| TRUE/FALSE Nitrous oxide has similar vasodilating effects to sevoflurane | FALSE |
| TRUE/FALSE Desflurane abolished autoregulation of cerebral blood flow at 0.7 MAC | FALSE At 0.7 MAC it is impaired but present At 1- 1.5MAC it is abolished |
| TRUE/FALSE Sevoflurane reduces pulmonary vascular resistance | TRUE N2O increases it. Others decrease it. |
| TRUE/FALSE Sevoflurane should not be used above 1.3 MAC because of the risk of coronary steal | FALSE Volatiles increase coronary blood flow many times beyond that of myocardial oxygen demand, thereby creating potential for steal. Coronary steal is diversion of blood from myocardial bed with limited or inadequate perfusion to a bed with more adequate perfusion (when autoregulation still present) |
| TRUE/FALSE Sevoflurane causes a dose dependent decrease in hepatic arterial blood flow | FALSE Decreases with halothane and enflurane only |

HEPATIC

Outline the clinical laboratory assessment of liver function

| An elevation in AST is related to zone 1 damage | There is more ALT than AST in zone 1 and more AST than ALT in zone 3 – You still get a rise in AST, but more of a rise in ALT – so I guess strictly speaking an isolated rise in AST is indicative of zone 3 not zone 1 damage |
| --- | --- |
| Hypoalbuminaemia is seen within 48 hours of a hepatic insult | F – Albumin has a half life of 20 days, and thus is not a good marker of hepatic injury |
| There will be hyperglycaemia in severe hepatic insufficiency | F – Impaired gluconeogenesis and glycogenolysis – so hypoglycaemia would be the result |
| Urea will be low in severe hepatic insufficiency | T – Impaired Urea cycle I’m assuming, thus an elevated ammonia and reduced urea |
| Biliary obstruction can cause a high INR | T – Also a lack of bile in the gut will impair Vit K absorption, impairing synthesis of coagulation factors |

Local anaesthetics

Niki

BT_RA 1.3 Discuss the pharmacology of local anaesthetic agents including:

· Mechanisms of action

· Comparative pharmacology of different agents

· Toxicity

· Use of adjuvant agents to enhance the quality or extend duration of block

· Pharmacokinetics of drugs administered in the epidural and subarachnoid space

12/12/16

| Duration of action of a local anaesthetic is primarily determined by the pKa of the agent TRUE/FALSE | False All LAs are weak bases with pKa higher than physiological pH - (exception: Benzocaine, pKa 3.5) - therefore all of them will have a higher percentage in the ionised form in blood than unionised - this affects their lipid solubility and therefore the speed at which they cross the cell membrane, potency and potentially latency (theoretically) The duration of action is primarily determined by their protein binding - affinity for protein binding correlates to affinity for proteins at receptor site within sodium channels, prolonging presence of LA at site of action. https://www.ncbi.nlm.nih.gov/pmc/articles/PMC1693664/ |
| --- | --- |
| Increasing the dose of local anaesthetic will increase the duration of action TRUE/FALSE | True The relationship between concentration and block onset is logarithmic, not linear; in other words, doubling the concentration of LA will only marginally speed up the onset of the block (although it will block the fibers more effectively and prolong the duration). The duration of nerve block anesthesia depends on the physical characteristics of the LA and the presence or absence of vasoconstrictors. The most important physical characteristic is lipid solubility. In general, LAs can be divided into three categories: short acting (e.g., 2-chloroprocaine, 45-90 minutes), intermediate duration (e.g., lidocaine, mepivacaine, 90-180 minutes), and long acting (e.g., bupivacaine, levobupivacaine, ropivacaine, 4-18 hours). The degree of block prolongation with the addition of a vasoconstrictor appears to be related to the intrinsic vasodilatory properties of the LA; the more intrinsic vasodilatory action the LA has, the more prolongation is achieved with addition of a vasoconstrictor. http://www.nysora.com/regional-anesthesia/foundations-of-ra/3492-local-anesthetics-clinical-pharmacology-and-rational-selection.html |
| Benzocaine is only suited to topical anaesthesia due to its lipophilicity TRUE/FALSE | ?True Benzocaine has a pKa 3.5 (weak base) - almost all unionised at physiological pH: makes it very "lipid soluble" at physiological pH, this wouldn't preclude IV/IM use - however physiochemical properties: would be difficult to store it a stable solution without either: 1. lowering the pH to 1.5 (couldn't inject this) or 2. making an emulsion (?injection of this into tissue: probably not pleasant, no point in using this IV: short DOA, very potent, probably toxic) |
| All local anaesthetics EXCEPT ropivicaine cause vasodilation TRUE/FALSE | False Cocaine also has strong vasoconstrictive properties. All have some degree of vasoactivity which is biphasic - vasoconstriction at low doses - then vasodilation at higher doses - ropivicaine: does become bisphasic as well but only at very high doses (stays in first phase/vasoconstrictor phase for higher doses than other) - except cocaine: vasoconstrictor at all doses due to inhibition of NAdr reuptake - David Olive's lecture slides |
| Adding bicarbonate to a local anaesthetic solution hastens the onset of action TRUE/FALSE | True Sodium bicarbonate (NaHCO3) is often added to LA. This increases the amount of drug in the base form, which slightly shortens the onset time. - due to being a weak base with pKa > phys pH http://www.nysora.com/regional-anesthesia/foundations-of-ra/3492-local-anesthetics-clinical-pharmacology-and-rational-selection.html |

13/12/16

| Early excitatory signs of neurotoxicity are due to activation of excitatory interneurons TRUE/FALSE | False Due to initial blocking of inhibitory interneurons. |
| --- | --- |
| High foetal plasma concentrations of local anesthetic are a result of higher α1-acid glycoprotein concentrations in the foetus TRUE/FALSE | False Foetal blood has a lower level of alpha1AGP = increased level of free drug Other causes of increased toxicity in foetus: 1. Ion trapping - foetal blood pH is lower than maternal pH - BIB their pKa - therefore the LA's in their unionised form diffuse across the placenta, become ionised in the foetal blood and then are "trapped" there. 2. Immature BBB - more able to diffuse across 3. Rapid heart rate - binding to the receptor is activity dependent: more will bind channels that are more active - bradycardia is bad in foetus and LA cardiac tox causes bradycardia |
| Methaemaglobinaemia from prilocaine toxicity results in a right shift of the oxygen haemaglobin dissociation curve TRUE/FALSE | FALSE Methb shifts to the left |
| All local anesthetics exert dose-dependent negative inotropic action on cardiac muscle TRUE/FALSE | True |
| The CNS effects of local anesthetics may contribute to the generation of arrhythmias TRUE/FALSE | True |

1/5/17

Discuss the factors affecting duration of action of a local anaesthetic block to a major peripheral nerve.

BT_RA 1.3

Lot of patient interest out there in you knowing this.

| Less lipid soluble drugs have a longer duration of action TRUE/FALSE | False More lipid solubility would lead to longer duration of action: "Although increasing lipid solubility may hasten axonal penetration, it may also result in increased uptake and sequestration of local anesthetics by myelin and other lipid-soluble perineural compartments, which results in a net effect of a decreased onset of action. The duration of action is prolonged as the sequestration of the more lipid- soluble local anesthetics within the myelin and surrounding perineural compartments leads to decreased vascular absorption and uptake, which provides a depot for slow release of the local anesthetic." - Evers pg 578 |
| --- | --- |
| Low protein binding causes a longer duration of action TRUE/FALSE | False Low protein binding causes a shorter duration of action. "It is likely that highly protein-bound local anesthetics are removed from the nerve at a decreased rate, resulting in slower uptake and absorption, which accounts for the increased duration of action. " - Evers pg 578 |
| Large molecules have a longer duration of action TRUE/FALSE | True Larger molecular weights of the aminoamides relates to increased lipophilicity. As per Evers above: increased lipophilicity leads to longer duration of action. |
| Less vascular areas have a longer duration of action TRUE/FALSE | True The duration of action of a local anaesthetic is proportional to the time the agent is in contact with the nerve axon. |
| In an elderly patient there will be a longer duration of action TRUE/FALSE | ?True Increased uptake of the LA from the injected site by blood stream causes decreased duration. - elderly would have poorer perfusion overall I would think so slower uptake and prolonged duration of action. |

Discuss the physiological consequences of total spinal anaesthesia caused by intrathecal administration of 20ml of  2% lignocaine at the L3/4 level. (Do not include management)

| This will cause bradycardia TRUE/FALSE | True - total SNS blockade - in article: all patients got atropine as a premed but still had lowered HR |
| --- | --- |
| The patient will have dilated gut TRUE/FALSE | False Ambu: gut's own ANS won't be affected. Also article mentions that without N2O and "SNS block of gut" gave superior closing conditions.. dilated would give poor closing conditions. |
| The patient will become hyperthermic TRUE/FALSE | False Article doesn't mention hyperthermia, but would make sense that they would become hypothermic. |
| The patient will have dilated pupils TRUE/FALSE | True Initially pupils constrict and are still reactive to light, then as all the cranial nerves blocked: dilated and non-reactive. |
| The patient will be unconscious TRUE/FALSE | True |

MEASUREMENT

Niki

This LO is massive, and covers essentially an entire textbook. For this reason we will return to it frequently.

BT_SQ 1.6  Describe the methods of measurement applicable to anaesthesia, including clinical utility, complications and sources of error in particular:

· SI units

· Measurement of volumes, flows, and pressures, including transducers.

· Measurement of blood pressure

· Measurement of cardiac output

· Measurement of temperature

· Oximetry

· Gas analysis, including capnography

· Methods used to measure respiratory function, including:

– Forced expiratory volume

–  Peak expiratory flow rate

–  Vital capacity

–  Flow-volume loops

–  Functional residual capacity and residual volume

28/11/16

| TRUE/FALSE A pulse oximeter radiating 2 wavelengths of light can only differentiate 2 different forms of Hb. | True |
| --- | --- |
| TRUE/FALSE Oxygenated haemoglobin absorbs light at a wavelength of 660nm. | True HbO2 - more at 940nm Hb - more at 660nm But they both also absorb light at each other's wavelengths (why you need to do a ratio of one to the other) |
| TRUE/FALSE The y axis on the plethysmograph is an estimate of arterial calibre and thus sympathetic tone. | True Lambert's law: the absorption of radiation as it passes through a substance increases exponentially as the distance it travels through the substance increases. Variability in the y-axis readings (independent of pulse waves) is due to increased thickness of the tissue due to arterial pressure increasing the calibre of the arteries. I think this is the maths behind using plethysmograph readings to estimate CO? |
| TRUE/FALSE Methaemoglobin is strongly absorbed at 660 and 940nm | True Methaemoglobin absorbs light very strongly at both 660nm and 940nm. Because the pulse oximeter measures the difference between the 2 wavelengths, it will affect both measurements, making the difference between then very small, even when there is a low concentration of metHb. When the ratio is the same, the pulse oximeter reads 85%. This is why methaemoglobin gives a reading tending towards 85% but not lower. |
| TRUE/FALSE An adult pulse oximeter cannot accurately read oxygen saturations when foetal haemoglobin is present. | False According to Aston equipment: not affected by the presence of different Hb types (HbF, HbA, HbS etc) |

29/11/16

Since you’ve already been reading about pulse oximeters we’ll stay on this topic.

| TRUE/FALSE Response time is faster when the oximeter is on the earlobe cf the finger | True |
| --- | --- |
| TRUE/FALSE Bilirubinaemia can result in a falsely low oxygen saturation with pulse oximetry | False |
| TRUE/FALSE The percentage of the signal which is pulsatile in finger pulse oximetry is approximately 80% | False About 2% - howequipmentworks.com |
| TRUE/FALSE Anaemia may cause under-reading of oxygen saturations with pulse oximetry | False Pulse oximetry doesn't differentiate the amount of Hb |
| TRUE/FALSE A pulse oximeter will detect a drop in oxygen tension from 600mmHg to 200mmHg | False Will only "detect" (estimate) PaO2 when it changes the SaO2 (Hb-O2 DC) |

30/11/16

Moving to measuring oxygen in its gaseous form…

| TRUE/FALSE An operating room paramagnetic analyser incorporates a pressure transducer | TRUE - alternating pressure at the transducer, the magnitude of which is a measure of the oxygen partial pressure in the sample gas. NOTE - old types had dumb bell |
| --- | --- |
| TRUE/FALSE Pressure exerted on the side of a tube decreases as flow rate increases | TRUE ? Bernoulli's principle? - Shrav's group weren't sure, I think it is true after reading this: http://physics.bu.edu/~duffy/py105/Bernoulli.html |
| TRUE/FALSE Nitric oxide at clinically used concentrations will falsely increase oxygen concentration in a paramagnetic analyser used in theatre | False Nitric oxide and O2 measured using PMGA - used in such low quantities 150ppm that doesn't affect the measurement - paramagnetic properties of NO are very small compared to O2 You could argue that it will increase the measured O2 conc but I think it would be so low that maybe it wouldn't even read (ie: less that 1%) |
| TRUE/FALSE Paramagnetic analysis degrades oxygen molecules into free radicals so the gas cannot be returned to the circuit | False Paramagnetic analysers just direct gases faster or slower down a tube, they do not change the molecules. Mass spectrometry "cracks" the molecules. |
| TRUE/FALSE Oxygen tension can also be measured with infrared analysis | False Infrared radiation is only absorbed by diatomic molecules (molecules that have two or more different elements in them): CO2, N2O, volatiles. It is not absorbed by molecules that only have one element in them: O2, N2, He |

"13/12/17" (belongs in a 20/12/16 but the blogger mucked up)

A contribution from an ex chair of the primary exam :

I was working in theatre with a registrar who had a previous degree in physics, and we were discussing the issues of accuracy with arterial monitors, in particular the concept of damping… Together we determined that this diagram probably illustrates some important concepts.

| TRUE/FALSE System A is an example of a system where the natural resonance frequency of the system is similar to that being measured. | FALSE It's underdamped. If the natural frequency of the system was equal to the fundamental frequency of the measured waveform, it would continue to oscilate indefinitely |
| --- | --- |
| TRUE/FALSE System B is the most ideal for a blood pressure measurement response because it provides the most accurate reading. | False Critically damped, (critically damped is the amount of damping where the signals returns to zero as fast as possible without overshooting) - would take too long to reach baseline each beat (may as well use a NIBP cuff) |
| TRUE/FALSE System C is an example of optimal damping as it provides a rapid response with minimal sacrificing of accuracy | True Optimal damping coefficient is 0.64 - compromise between overshooting and not taking too long to get back to zero, in physiological circuits this is 0.64 or 64% of critical damping. |
| TRUE/FALSE In all systems the eventual measured pressure will be accurate | True When damping coefficient =0 in a perfectly frictionless circuit, the measurement will never reach a baseline (will just continue to oscillate at the same amplitude). In reality (outside physicists' imaginations) this wouldn't be possible because there would be some resistance in the circuit to make this happen. Apparently however the MAP will be accurate. |
| TRUE/FALSE Damping is a reduction in the amplitude of an oscillation as a result of energy being drained from the system to overcome frictional or other resistive forces. | True |

15/12/16

Perhaps today’s topic, dynamic airway closure, will be of more interest as it is one that candidates really struggle with in vivas.

BT_SQ1.6 Describe the methods of measurement applicable to anaesthesia, including clinical utility, complications and sources of error in particular:

· Methods used to measure respiratory function, including:

– Forced expiratory volume

– Peak expiratory flow rate

– Vital capacity

– Flow-volume loops

– Functional residual capacity and residual volume

West describes the topic well in his book, where the following diagram is taken from. The red circle shows the net pressure gradient between the intrapleural space and the airway.

| Dynamic airways closure may occur during normal tidal breathing TRUE/FALSE | TRUE |
| --- | --- |
| Dynamic airway closure accounts for the effort dependent portion of the expiratory limb of the flow-volume loop TRUE/FALSE | FALSE Dynamic airway closure accounts for effort INDEPENDENT component Starlings resistor - Alveolar pressure - INTRPLEURAL pressure (not mouth) |
| During forced expiration, positive pressure generated will be transmitted equally across the respiratory system TRUE/FALSE | False Decreases along length of airway |
| The trachea is never subject to dynamic airway closure TRUE/FALSE | False Tracheal tug in babies Excessive dynamic airway collapse (EDAC) defines the pathological collapse and narrowing of the airway lumen by >50%, which is entirely due to the laxity of the posterior wall membrane : Pierdonato Bruno1 |
| During the effort independent part of an expiratory flow volume loop, maximum air flow rate is determined by lung volume TRUE/FALSE | True Maximal flow decreases with lung volume - FIG 7.16 WEST |

20/12/16

Once again I was with my registrar with the expert knowledge of physics waiting for the cardiothoracic registrar to take down the mammary.  It was as though time stood still.   We were both looking at the clock, and reminiscing on the wonderful mechanics of (non digital) clocks.  She commented to me in passing “I do miss the slow natural frequency of the pendulum of a grandfather clock” which made me consider the fast swinging pendulum of a cuckoo clock.   Needless to say, my mind turned to the concept of natural resonance frequencies in invasive pressure monitoring systems, and I thought back to the days of my music lessons…

| TRUE/FALSE The natural resonant frequency of a system is proportional to the stiffness or tension in the system, and inversely proportional to the mass. | TRUE Hence - decreased compliance and increased density (ie. Clots) - decr natural frequency of arterial set up Also Increased length decr nf |
| --- | --- |

| TRUE/FALSE As in tightening a violin or guitar string, increasing the stiffness or tension will lead to an increase in the natural resonant frequency (a higher note on the instrument) | True |
| --- | --- |
| TRUE/FALSE Like the pendulum of a grandfather clock being slower than that of a cuckoo clock on the wall, the pulmonary artery tracing on the monitor is not as good as the arterial system, as the pulmonary artery system has a much longer system and as such more mass and a lower natural frequency | ?TRUE • Has a lower natural frequency because LONGER tubing and hence increased mass • Therefore high frequency artefacts Above as per Shrav's group, I agree: https://books.google.com.au/books?id=45DKiUj1hLUC&pg=PA206&lpg=PA206&dq=natural+frequency+of+pulmonary+artery+trace&source=bl&ots=UOFpYsCxIv&sig=T68cxhHPIIA_uC7K_2n5DMURl2M&hl=en&sa=X&ved=0ahUKEwj0t9br57fUAhXHS7wKHZcRC_0Q6AEIKDAA#v=onepage&q=natural%20frequency%20of%20pulmonary%20artery%20trace&f=false |
| TRUE/FALSE The ideal system for an arterial monitoring system has a large length and very stiff tubing to ensure that its natural frequency is close to the frequency of the system being monitored | False You don't want the frequency of the system close to the natural frequency, otherwise you will get resonance pg 200 Aston |
| TRUE/FALSE The ideal frequency for a pressure monitoring system is determined by the pressure range being measured, rather than by the frequency of the system. | FALSE The frequency characteristics are independent of the pressures measured. The natural frequency (resonant frequency) of the measuring system should be at least 8 times the fundamental frequency of that being measured. 180bpm = 3 hz * 8 = 24hz |

22/12/16

BT_SQ 1.6 Describe the methods of measurement applicable to anaesthesia, including clinical utility, complications and sources of error in particular:

· Gas analysis, including capnography

| TRUE/FALSE The 90-95% response time for a CO2 analyser should be less than 150ms | ?True This article says 50-600ms is used clinically, but I would think 150ms would be better, especially high RR. |
| --- | --- |
| TRUE/FALSE Volatile agents can be distinguished from each other by measuring infrared absorbance at 3.3µm | False The absorption spectrum for volatiles is 8-13 μm range - although one graph I found showed desflurane with an absorption spectrum around 3, still I would be more likely to trust this source: https://www.ncbi.nlm.nih.gov/pmc/articles/PMC3821265/ |
| TRUE/FALSE Collision broadening means that the absorption peak for CO2 at 4.3µm is made wider in the presence of Nitrous Oxide | True |
| TRUE/FALSE Infrared analysers measure gas concentration rather than partial pressure | False Infrared analysers measure the partial pressure. - pressure usually displayed as continuous partial pressure vs time (capnograph) |
| TRUE/FALSE Water is a powerful absorber of infrared light | True Gas has to be dried before being analysed - passed through Nafion tubing (modified form of teflon) that is selectively and highly permeable to water - also in above book for Q1 |
| Follow on questions: 1. Why is the 90% response time important in a CO2 analyser? The answer is related to what you have learned about requirements for invasive pressure monitoring | 90% response time: the delay between sampling gas for CO2 measurement and the display of the measurement. Components of response time: - transit time - rise time Important because may affect: - changes capnograph shape - delay in capnograph compared reading (problem in critical intubations) - underestimation of ETCO2 (particularly in children due to rapid RR) - reduce the slope of phase II resulting in an underestimation of anatomical dead space.3,5 http://www.capnography.com/new/physics/chemical-method-of-co2-measurement?id=64 |
| Follow on questions: What is the device in the picture below? What is it used for? How does it work? | ? I don't know what this is, anyone else? |

BT_GS 1.51 Describe the concept of depth of anaesthesia and how this may be monitored

10/1/17

| TRUE/FALSE The EEG of a patient when anaesthetised has a smaller amplitude than when they are awake | False Millers: |
| --- | --- |
| TRUE/FALSE Anaesthesia with ketamine produces a different pattern of brain EEG compared to anaesthesia with volatile agents | True |
| TRUE/FALSE If two patients have the same BIS number, then they are anaesthetised at the same depth of anaesthesia | ?False I feel like this is one of those all or nothing statements that just can't be true.. |
| TRUE/FALSE Administration of a muscle relaxant will reduce the BIS index if EMG is present | True |
| TRUE/FALSE The Entropy monitor measures the effect of anaesthetic drugs on the brain by calculating the randomness of the EEG | True |

BT_GS 1.55 Describe the concept of depth of neuromuscular blockade and explain the use of neuromuscular monitoring

2/2/17

| TRUE/FALSE Response to repeated single twitches at 1Hz is greater than at 0.1Hz | False At frequencies above 0.15Hz, the response will gradually decrease and settle at a lower level http://www.ld99.com/reference/ notes/text/Neuromuscular_monitoring.html Miller agrees |
| --- | --- |
| TRUE/FALSE In the late phase of recovery after muscle relaxant admnistration, tetanic stimulation can cause lasting antagonism of neuromuscular blockade | TRUE Reference as above |
| TRUE/FALSE The post tetanic count at which the first twitch on the train of four (TOF) appears is similar for both atracurium and cisatracurium | True - miller |
| TRUE/FALSE To completely prevent the bucking response to carinal stimulation the post tetanic count needs to be zero | TRUE - Miller fig 53.6 |
| TRUE/FALSE If no fade is felt with dual burst stimulation the train of four ratio is above 0.7 | ?True Double burst was developed because distinguishing face in TO4 was very difficult when ratio was >0.7 (supposedly would be able to distinguish), so if you can't determine fade in double burst then probably TO4 is >0.7 |

21/2/17

BT_SQ 1.6 Describe the methods of measurement applicable to anaesthesia, including clinical utility, complications and sources of error in particular

· SI units

· Measurement of volumes, flows, and pressures, including transducers.

· Measurement of blood pressure

· Measurement of cardiac output

· Measurement of temperature

· Oximetry

· Gas analysis, including capnography

· Methods used to measure respiratory function, including:

– Forced expiratory volume

–  Peak expiratory flow rate

–  Vital capacity

–  Flow-volume loops

–  Functional residual capacity and residual volume

| TRUE/FALSE If exhaled gas is not warmed to patient temperature in a pneumotachograph, volume will be underestimated | ?False Pneumotachographs make the flow laminar, then measure the differential pressure. - if the flow becomes turbulent: error in reading Viscosity varies with temperature - increased temp = increased viscosity - decreased temp = decreases viscosity Therefore: lower temperature = lower viscosity in the pneumotachograph if it is not heated. Reynold's number = vpd/viscosity - will be higher with lower viscosity - flow may become turbulent: this would just make the calculations inaccurate because the pneumotachograph is calculating the flow with the Hagan-Poiseuille equation rather than the turbulent flow equation. ALSO If the gas is colder, the viscosity will decreases, causing an increased laminar flow as per Hagan-Poiseuille. - this would cause it to OVERestimate. |
| --- | --- |
| TRUE/FALSE Volume is the area under a flow/time curve | TRUE. Flow = Volume/Time, therefore Volume = Time *Flow = AUC |
| TRUE/FALSE A pneumotachograph calculates flow from a known resistance and a measured pressure difference | True Pneumotachograph is a constant orifice (constant resistance), variable pressure flowmeter. |
| TRUE/FALSE A pneumotachograph uses the hydraulic version of Ohms Law | True |
| TRUE/FALSE A pneumotachograph measures flow accurately only when it is turbulent | False |
| This is a Dräger flow sensor from one of the limbs of a circle circuit. If you look closely you can see a fine wire between the top two prongs. The wire between the lower two prongs isn’t shown so clearly. What principle is this flow sensor using? | Thermal anemometry: hot wire. - is the most common method used to measure instantaneous fluid velocity. The technique depends on the convective heat loss to the surrounding fluid from an electrically heated sensing element or probe. If only the fluid velocity varies, then the heat loss can be interpreted as a measure of that variable. - the heat change relates to increased electrical resistance within the wire which can be measured (via wheatstone bridge) and then flow calculated. http://www-g.eng.cam.ac.uk/whittle/current-research/hph/hot-wire/hot-wire.html |
| Here are two different views of the sensor that modern GE machines use for measuring flow in a circle. The two tubes are hollow, and you can see on the photo on the left that they are open to the circuit. What principle is this flow sensor using? How do you think that it works? | A pneumotachometer: fixed orifice, differential pressure. - uses a restrictor in the gas flow passage to create a pressure drop that can be sensed by a differential pressure transducer. http://www.apsf.org/newsletters/html/2008/spring/08_dearsirs.htm |

26/5/17

Recently I was anaesthetising an adult with a congenital syndrome. I was quite worried about the airway—but in the end it wasn’t that which caught me out. She had no congenital heart disease, but had a pericardial effusion drained a few years previously. I was quite sparing with the induction agents as I wanted to maintain spontaneous respiration, but nonetheless…

About 5 minutes after induction, I noticed the blood pressure was 54/28.

BT_SQ 1.6

| T/F At low levels of blood pressure, the NIBP tends to give spuriously low values. | True |
| --- | --- |
| T/F The most accurate component of the NIBP is the mean. | True In oscillometric device: MAP is the largest amplitude of oscillations. |
| At the same time, her saturation dropped to 88, even though she was breathing 100% oxygen. The pleth had a good volume and looked normal. BT_SQ 1.6, BT_PO 1.29 T/F The fall in SpO2 was most likely to be artifactual. | True It is likely due to the low blood pressure. - plethysmography works off the difference in diameter between diastolic and systolic pulsations, the difference is already very low so if the pulsation is weak and causes an even smaller increase in diameter: pulse oximeter is inaccurate |
| I gave three doses of 1mg metaraminol, but, although the saturation improved, the blood pressure remained in the low 70s. Heart rate was in the 40s. Worried that I might see another fall in saturation I decided to run a noradrenaline infusion. BT_PO 1.52 In such a situation, the most appropriate vasoactive agent would be: a) Ephedrine b) Metaraminol c) Adrenaline d) Noradrenaline e) Isoprenaline | a This is what I would use, most readily available, longer duration of action as a bolus, has both alpha 2 and beta 1 agonist actions, safer peripherally, larger margin of error for titrating. Could use adrenaline infusion if you wanted to but don't have central access (although it can be run peripherally) and very small amounts would only give you beta 1, when you also need alpha 2 action. |
| After a 20µg bolus dose of noradrenaline, the heart rate dropped to 28. T/F The most likely cause of the fall in heart rate is alpha 1 receptor agonism in the SA node. I found out, after the (otherwise uneventful) operation, that she normally has quite a low blood pressure. A good reminder that, when having trouble with anaesthesia, one should first look to the proximal end of the needle. | False Reflex bradycardia |

These questions relate to ROTEM which seems to be the favourite viscoelastic assay in use at present. Even my humble little hospital has one of these things.

29/5/17

| Q. It takes about an hour to provide clinically useful information. TRUE/ FALSE | False The clot formation information should be ready earlier, fibrinolysis measurement may take about an hour. |
| --- | --- |
| Q. Will reliably detect platelet dysfunction as a result of clopidogrel therapy. TRUE/ FALSE | False Different additives to the ROTEM cup will be able to detect the effect of platelets on clotting - FIBTEM reagent contains ctyochalasin D (platelet inhibitor) so clot will be independent of platelet function - EXTEM reagent doesn't inhibit platelets: so difference between EXTEM and FIBTEM is the clotting action due to platelets However: you can't tell the difference between platelet dysfunction due to clopidogrel and that due to aspirin if patient is on both aspirin and clopidogrel, just overall platelet dysfunction TEG can tell the difference with difference reagents added - PlateletMapping system |
| Q. Reliably detects the presence of heparin therapy. TRUE/ FALSE | True Reagents: - HEPTEM: contains hepatinase - compared to INTEM - can detect coagulation inhibition due to heparin |
| Q. Can detect primary and delayed fibrinolysis. TRUE/ FALSE | True LY30 and LY60 - measures of clot strength at 30 and 60 minutes - gives indication of how quickly fibrinolysis is occuring |
| Q. Will be abnormal if the patient is hypothermic. TRUE/ FALSE | False You can alter the operating temperature of the machine so that it's the same as the patient - by comparing results of altered temp to 37C: can work out the effect temp is having on clotting - so would only be abnormal if you didn't know the patient was hypothermic |

METABOLIC

Niki

Whilst fasting for a minor surgical procedure last week, I had plenty of time to ponder this topic…

BT_PO 1.83 Describe the physiological consequences of starvation

| Prolonged starvation reduces immune function TRUE/FALSE | ?True |
| --- | --- |
| Brain and nerves, renal medulla, red blood cells are obligate glucose consumers TRUE/FALSE | FALSE Brain can use ketone oxidation Power and Kam, p 375 Renal medulla and RBCs use glucose, which can be synthesized via gluconeogenesis but not sure if renal medulla is obligate - pretty sure RBCs are only obligate glucose users |
| Adaptive reponses to starvation aim for conservation of energy and protein TRUE/FALSE | True Plasma protein levels are maintained during starvation until body protein stores are markedly depleted. (Ganong pg 565) T3 levels decreased: conserve calories by restricting tissue metabolism. (Ganong pg 345) |
| Muscle glycogen is freely available as a source of blood glucose TRUE/FALSE | False Can't be mobilised as source of blood glucose, only glycogen in the liver is able to be mobilised to the blood. Muscle glycogen is only available for muscle metabolism |
| During starvation, the gastrointestinal tract has an enhanced ability to digest food TRUE/FALSE | True Evidence suggests that the peptidase activities of the brush border and the mucosal cell cytoplasm are increased by resection of part of the ileum and that they are indepen- dently altered in starvation. (Ganong 482/3) |

I visited Canberra towards the end of last year to watch one of my children run. In my (large) amount of free time, I visited the National Portrait Gallery, where I happened upon this portrait of Derek Denton, the Australian scientist who discovered the mechanism of thirst.

BT_PO 1.76  Describe the regulation of osmolality

| The factors that regulate vasopressin secretion also regulate thirst TRUE/FALSE | True Angiotensin II and SNS |
| --- | --- |
| Vasopressin is synthesised in the posterior pituitary TRUE/FALSE | False Synthesised in the hypothalamus, transported to posterior pituitary where it is release. |
| Most of plasma’s osmolality is due to Na+ and its accompanying ions, CL- and HCO3- TRUE/FALSE | True |
| Hypovolaemia stimulates thirst via angiotensin II acting at sites outside the blood brain barrier TRUE/FALSE | True |
| Aldosterone is responsible for regulating plasma osmolality TRUE/FALSE | True |

NERVOUS

Niki

This is the first of three posts on the EEG – 1) the basic physiology, 2) how drugs affect the EEG, and 3) quantitative EEG monitors.

Inferring the state of consciousness by analysing electrical voltages on the surface of the forehead may seem a bit like trying to see who is winning the football by holding a voltmeter up to the TV screen. However, as reversible obliteration of consciousness is our core business, and the electroencephalogram (EEG) is one of the few ways to observe the effect of drugs on the brain, it is important to understand some basics of the EEG, in part so that the various claims made for quantitative EEG devices can be assessed critically.

Useful resources include the 8th Ed of Miller, especially Chapter 17, which is available online from the ANZCA website.

| T / F The frontal EEG is a mixture of electrical signals derived from the cerebral cortex as well as sub-thalamic structures and the limbic system. | True |
| --- | --- |
| T / F An “activated” EEG means that the amplitude of the EEG waveform is reduced. | False When events occur that lead the brain to produce higher frequencies and larger amplitudes, the EEG is described as activated, and when slower frequencies are produced (theta = 4 to 7 Hz, and delta = <4 Hz), the EEG is said to be depressed. Miller 8ed, Neurologic Monitoring chapter. |
| T / F The amplitude of an awake EEG is about the same as the p-wave on a standard ECG. | ?False Amplitude is the size, or voltage, of the recorded signal and ranges commonly from 5 to 500 μV (versus 1 to 2 mV for the electrocardiogram signal). -Miller |
| T / F The amplitude of the EEG decreases with age. | True Because neurones are lost -Miller |
| T / F “Burst suppression” is defined as periods of electrical activity alternating with periods of isoelectric EEG. | True Burst suppression is periods of suppression interspersed with periods of activity. Produced by - volatiles MAC >1.5 (limited by toxicity) - barbiturates: high doses - etomidate: high doses - propofol: high doses Not with - N2O (alone) - ketamine - benzodiazepines - opioids - dexmed |

NEUROMUSCULAR AGENTS

Niki

Discuss the potential adverse effects of suxamethonium

BT_GS 1.38

This is a drug with a cornucopia of adverse effects, know them so you know when not to use the drug, and what to be alert for every time you do use it.

| Hyperkalaemia is more likely with a repeat dose TRUE/FALSE | True |
| --- | --- |
| Bradycardia is more likely with a repeat dose TRUE/FALSE | True |
| Prolonged paralysis is more common in South East Asians than Caucasians TRUE/FALSE | True The genes for the inheritance of plasma cholinesterase are autosomal. There are several variations from the normal enzyme E1U. The most common of these is E1a. This abnormal gene is carried by 4% of the Caucasian population. This figure is higher in Asians and those from the Middle East and lower in Africans 1,2. https://www.google.com.au/url?sa=t&rct=j&q=&esrc=s&source=web&cd=1&ved=0ahUKEwjkyoaG38PUAhVBmJQKHeRnAsQQFggqMAA&url=http%3A%2F%2Fwww.e-safe-anaesthesia.org%2Fe_library%2F13%2FSuxamethonium_apnoea_Update_2003.pdf&usg=AFQjCNHi4kP9CtGLJ-W95p13oV7a3kZPRQ&sig2=aLLDKnnbe1UCGQgnLBbo0g |
| Myalgia is more likely in a patient who has a spinal cord injury TRUE/FALSE | ?False High K+ level are more likely but I would think if you can't feel your legs you are unlikely to get myalgia... |
| Masseter spasm can be an early sign of MH TRUE/FALSE | True |

WEDNESDAYS

Niki

31/5/17

| Regarding PGF2 alpha: Q. It increases uterine tone. TRUE/ FALSE | True Used for PPH due to uterine atony that is not responsive to oxytocin, ergometrine, misoprostol. - all on RWH management of PPH flowchart. But: relaxes cervix |
| --- | --- |
| Regarding PGF2 alpha: Q. It can cause bronchoconstriction. TRUE/ FALSE | True PGF2 alpha is a potent bronchoconstrictor. |
| Regarding PGF2 alpha: Q. It reliably causes systemic hypertension. TRUE/ FALSE 31/5/17 | False Can cause hypertension or hypotension. |
| Regarding PGF2 alpha: Q. Can cause a low grade fever. TRUE/ FALSE 31/5/17 | True In 1-5% of cases |
| Regarding PGF2 alpha: Q. Is administered in small aliquots intravenously. TRUE/ FALSE | False Can be given IV as an infusion for induction of labour. Can be given into amnionic fluid for late term abortion. |

7/6/17

Describe the utero-placental circulation and the principles of placental physiology as related to placental gas exchange and regulation of placental blood flow.

| The primary limitation to carbon dioxide transfer across the placenta is blood flow. TRUE/FALSE 7/6/17 | True |
| --- | --- |
| Maternal-fetal exchange of most drugs and other substances occurs primarily by diffusion TRUE/FALSE 7/6/17 | True Lipid soluble substances will diffuse across. There is some active transport for larger and polar molecules. |
| Foetal O 2 saturation does not exceed 40% even with 100% O 2 delivery to the mother. TRUE/FALSE 7/6/17 | False Which saturation are they talking about? - umbilical vein: 80% - RA: 67% - LA: 62% - pre-ductal descending aorta: 62% - post ductal descending aorta: 58% All of the saturations are above 40% |
| The placenta synthesises progesterone from cholesterol. TRUE/FALSE 7/6/17 | True "One of the essential roles of the human placenta is to produce the steroid hormone progesterone, which is required for the maintenance of pregnancy. The rate-determining step of placental progesterone synthesis is the conversion of cholesterol to pregnenolone by cytochrome P450scc (CYP11A1) in placental mitochondria in a reaction requiring electrons delivered via adrenodoxin reductase and adrenodoxin. Pregnenolone is converted to progesterone by type 1 3beta-hydroxysteroid dehydrogenase located in the mitochondrion." |
| O2 delivery to the fetus is facilitated primarily because the fetal oxyhemoglobin dissociation curve is righ-shifted. TRUE/FALSE. 7/6/17 | False Foetal O2-HB DC is left shifted to take up more O2 due to foetal FHb having greater affinity for O2 than adult alphaHb Foetal P50 is 19.7mmHg Maternal P50 is 27mmHg |

14/6/17

| Fibrinogen: Q. Is designated by the Roman numeral II. TRUE/ FALSE | False Fibrinogen is clotting factor I Prothrombin is II |
| --- | --- |
| Fibrinogen: Q. Is the predominant clotting factor found in Prothrombinex. TRUE/ FALSE | False Fibrinogen is in cryoprecipitate Not in Prothrombinex - prothrombinex contains Vit K dependent clotting factors: II, IX and X in Aus, in other places also contains VII |
| Fibrinogen: Q. Is the predominant clotting factor found in the circulation. TRUE/ FALSE | True Comprises about 7% of plasma protein. |
| Fibrinogen: Q. Has a reduced concentration in the pregnant individual. TRUE/ FALSE | False Fibrinogen levels increase in pregnancy to an average of 4.5 g/l, compared to an average of 3 g/l in non-pregnant people. Also you should probably aim for a higher fibrinogen level in PPH (resus guidelines). |
| Fibrinogen: Q. Is a Vitamin K dependent clotting factor. TRUE/ FALSE | False |

21/6/17

BT_GS 1.16 Describe alterations to drug response due to obesity

BT_PO 1.16  Describe alterations to drug response due to physiological change with particular reference to the elderly

This is a complicated issue [I’m giving you a free true statement]

| It is appropriate to dose muscle relaxants, such as vecuronium and rocuronium, based on ideal body weight (IBW) TRUE/FALSE | False (Non-depolarising) "Neuromuscular blockers should be administered to obese patients on the basis of about 20% more than lean body mass rather than on their actual body weight412 to ensure that these patients are not receiving relative overdoses.": Millers Elderly patients: increased body fat, decreased lean body mass - onset of NMB not affected but clearance affected by decreased renal and hepatic function: will last |
| --- | --- |
| When using propofol for maintenance of anaesthesia, calculate the infusion rate based on total body weight (TBW) TRUE/FALSE | ?True Can do either TBW or IBW + (0.4 x extra body weight) - if use TBW: probably overdosing with the deleterious CVS effects increased but clearance is the same. |
| The increased cardiac output seen in morbid obesity, will hasten recovery from volatile anaesthesia TRUE/FALSE | ?False I can't find anything in the texts about cardiac output and washout. The problem is washout initially also includes ongoing uptake by fat and muscle groups, so increased CO might increase that portion, then when venous partial pressure is lower than the fat and muscle increased CO might slow washout because these groups will start offloading their volatile into the blood. I guess this would be exaggerated in morbidly obese patients. The other bit is that the muscle and fat continue to take up volatile at a level usually below the MAC awake level and then start to offload before the patient wakes up, so I think they actually slow wake up in the end... Please comment if you know! |
| Plasma levels of pseudocholinesterase are increased with morbid obesity TRUE/FALSE | True |
| Suxamethonium doses should be based on TBW TRUE/FALSE | True Exception to the IBW + 20% rule |

BT_PO 1.71  Explain the effects of anaesthesia on renal function

28/6/17

| Any anaesthetic agent which results in a reduction of blood pressure is likely to reduce GFR TRUE/FALSE | True GFR = Kf[Pgc - Pb] + [πb - πgc] Direct effect on Pgc Decreased MAP = increased SNS Increased SNS causes decreased renal blood flow and GFR and increased Na+ and water conservation |
| --- | --- |
| Attenuation of the stress response to surgery is renal protective TRUE/FALSE | True As above: decreases sympathetic stimulation |
| Volatile anaesthetic agents may provide protection against ischaemia- reperfusion injury of the kidney TRUE/FALSE | True Recent studies suggest that modern halogenated volatile anesthetics induce potent anti-inflammatory, antinecrotic, and antiapoptotic effects that protect against ischemic AKI https://www.ncbi.nlm.nih.gov/pmc/articles/PMC4005317/ |
| IPPV improves renal blood flow TRUE/FALSE | False Decreases renal blood flow: - decreased MAP and CO -> increased SNS -> RAAS stimulation - decreased venous return -> decreased ANP -> less diuresis - decreased venous return -> venous congestion http://www.frca.co.uk/article.aspx?articleid=100426 |
| Metabolic acidosis increases the kidneys’ vulnerability to nephrotoxins TRUE/FALSE | True Metabolic acidosis or alkalosis may lead to alkaline or acidic urine. Alkaline and acid urine lead to increased sensitivity to nephrotoxics. ALSO: hyperchloraemia increases sensitivity: causes an acidosis, also one that anaesthetists will commonly inflict with N/saline. http://cjasn.asnjournals.org/content/4/7/1275.full#T2 |

| A TCI using the Marsh algorithm will give the same dose of propofol to an eighty year old and a twenty year old patient of the same weight. | T Age not in algorithm |
| --- | --- |
| TCI can be used for morbidly obese patients. | T – though not accurate |
| The Minto algorithm for Remifentanil was devised by an Australian anaesthetist. | TRUE (Sydney) |
| Plasma or effect site TCI can be used effectively for the Schnider algorithm. | TRUE |
| The James equations are used to calculate LBM in the Minto and Schnider algorithms. | TRUE |

Regarding PGF2 alpha:

| It increases uterine tone. | T |
| --- | --- |
| It can cause bronchoconstriction. | T |
| It reliably causes systemic hypertension. | T |
| Can cause a low grade fever. | T |
| Is administered in small aliquots intravenously. | FALSE – carboprost IM |

The formulation currently being used in my hospital is sourced from Russia. No one in the Department can

read Russian.

Keo

| Is the rate constant that describes transfer of drug from the central compartment to the effect site. | FALSE - OUT |
| --- | --- |
| Is the rate constant for elimination of drug from the effect site. | T |
| Is directly proportional to the t1/2keo. | F |
| Can be measured directly using frequent blood sampling. | FALSE – not true compartment |
| Has units of inverse time. | TRUE |

Recently I was anaesthetising an adult with a congenital syndrome. I was quite worried about the airway—but in the end it wasn’t that which caught me out. She had no congenital heart disease, but had a pericardial effusion drained a few years previously. I was quite sparing with the induction agents as I wanted to maintain spontaneous respiration, but nonetheless…

About 5 minutes after induction, I noticed the blood pressure was 54/28.

BT_SQ 1.6

| At low levels of blood pressure, the NIBP tends to give spuriously low values. | False Overestimate at low |
| --- | --- |
| The most accurate component of the NIBP is the mean. | T |

At the same time, her saturation dropped to 88, even though she was breathing 100% oxygen. The pleth had a good volume and looked normal.

BT_SQ 1.6, BT_PO 1.29

| The fall in SpO2 was most likely to be artifactual. | F - hypoperfusion |
| --- | --- |

I gave three doses of 1mg metaraminol, but, although the saturation improved, the blood pressure remained in the low 70s. Heart rate was in the 40s. Worried that I might see another fall in saturation I decided to run a noradrenaline infusion.

BT_PO 1.52

| In such a situation, the most appropriate vasoactive agent would be: a) Ephedrine b) Metaraminol c) Adrenaline d) Noradrenaline e) Isoprenaline | D |
| --- | --- |

After a 20µg bolus dose of noradrenaline, the heart rate dropped to 28.

| The most likely cause of the fall in heart rate is alpha 1 receptor agonism in the SA node. | T |
| --- | --- |

I found out, after the (otherwise uneventful) operation, that she normally has quite a low blood pressure. A good reminder that, when having trouble with anaesthesia, one should first look to the proximal end of the needle.

Tranexamic Acid:

| Is contraindicated in individuals who are colour blind | T |
| --- | --- |
| Is a lysine analogue. | T |
| Inhibits the activity of plasmin. | F Inhibit lysine binding site on Plasminogen to convert to plasmin |
| Should not be given as an IV push. | T |
| Can cause seizures. | T |

Entonox which, as I’m sure you all know, is a 50:50 mixture of oxygen and nitrous oxide.

| An Entonox cylinder contains a mixture of liquid and vapour. | T |
| --- | --- |
| An Entonox cylinder is coloured French blue and white. | T |
| The Poynting effect refers to how interactions between two different gases leads to a change in their physical properties. | T |
| Entonox provides about 0.5 MAC. | T – if breathe pure N2O 98% v/v |
| Can cause megaloblastic anaemia. | T |

| LMWH does not cause HITTS. | T Think this might be false? https://www.ncbi.nlm.nih.gov/pubmed/22972111 Looks like less risk of HITTS with LMWH verses UFH but I think it is still a risk.. |
| --- | --- |
| High dose UFH is characterized by 0 order pharmacokinetics. | T |
| All heparins greatly increase the activity of Antithrombin | T |
| Heparins are recombinant products. | T |
| Recent administration of 5000U heparin sc is a contraindication to neuraxial blockade. | 12 hrs |

Compare and contrast low-molecular-weight heparin (LMWH) and unfractionated heparin (UFH).

BT_PO 1.118 These are drugs that are commonly administered to our patients pre-operatively and can impact upon our procedural decisions.

| LMWH only inhibits factor XI | F |
| --- | --- |
| High dose UFH can inhibit platelet aggregation | T |
| Only UFH can be administered IV | F |
| Only UFH prolongs APTT | F |
| LMWH has less predictable pharmacokinetics | F - MORE |

Discuss the potential adverse effects of suxamethonium

BT_GS 1.38

This is a drug with a cornucopia of adverse effects, know them so you know when not to use the drug, and what to be alert for every time you do use it.

| Hyperkalaemia is more likely with a repeat dose | T |
| --- | --- |
| Bradycardia is more likely with a repeat dose | T |
| Prolonged paralysis is more common in South East Asians than Caucasians | T |
| Myalgia is more likely in a patient who has a spinal cord injury | Young female |
| Masseter spasm can be an early sign of MH | TRUE |

Describe the advantages and disadvantages of using nitrous oxide as part of a general anaesthetic (well, this wasn’t exactly the question, I have corrected the spelling error that was on the paper :P)

BT_GS 1.27

This was a commonly used component of anaesthesia up until the late 90s. Can you sum up why it was used almost universally then, why many anaesthetists don’t use it routinely now, and where you think it should fit into your anaesthetic practice?

| Nitrous oxide is more soluble in blood than desflurane | T BGPC 0.47 Des – 0.42 |
| --- | --- |
| Nitrous oxide is more soluble in blood than nitrogen | T |
| Nitrous oxide undergoes approximately 10% hepatic metabolism | F Minimal as per PI |
| Nitrous oxide increases plasma homocysteine levels | T |
| Nitrous oxide is an analgesic agent | T |

Propofol and remifentanil target controlled infusions are often given together as a total intravenous anaesthesia technique. Discuss pharmacological reasons why this is a useful combination.

BT_GS 1.59    BT_GS 1.53    BT_GS 1.41

A practical pharmacology question on a common drug combination. Before setting out to write a model answer try asking yourself first what are the clinical reasons you use this combination.

| There are significant pharmacokinetic interactions between these drugs | F |
| --- | --- |
| There are significant pharmacodynamic interactions between these drugs | F |
| Both drugs have a rapid offset | T |
| Adding remifentanil to propofol can lead to more stable haemodynamics | T |
| Can be used in patients susceptible to malignant hyperthermia | T |

As promised…

SS_PA 1.51  : Describe how the pharmacokinetics of drugs commonly used in anaesthesia in neonates and children differ from adults and the implications for anaesthesia

| Neonates require larger doses of neuromuscular blockers per kg than adults | f |
| --- | --- |
| Neonates require larger doses of remifentanil per kg than adults | f |
| Neonates require a larger induction dose of thiopentone per kg than adults | f |
| Higher doses of EMLA can be more safely used in neonates than older children | f |
| Surgical stress decreases the concentration of alpha 1 acid glycoprotein | t |

SS_PA 1.51  : Describe how the pharmacokinetics of drugs commonly used in anaesthesia in neonates and children differ from adults and the implications for anaesthesia

| Nasal midazolam tastes good | f |
| --- | --- |
| Oral medications are absorbed slowly in infants due to decreased gastric emptying and intestinal motility | t |
| Gastric pH is lower in infants than in adults | F – PH 7 -8 |
| The solubility coefficients of an inhaled anaesthetic will determine its volume of distribution | t |

Drug Variability

BT_PM 1.15  Discuss the pharmacokinetic and clinical implications of different routes of administration for commonly used opioids, including the oral, transdermal, subcutaneous, intramuscular and intravenous routes, and with particular reference to fentanyl, morphine, methadone, tramadol and codeine

SS_OB 1.13 Explain the factors that influence the transfer of drugs across the placenta

| Ionised drugs are more likely to cross the placenta compared with non-ionised drugs | F |
| --- | --- |
| Heparin is safe to use in pregnancy because its large molecular size prevents it crossing the placenta | T |
| Weakly basic drugs, with a pKa less than 7.4, may become concentrated in the fetal compartment secondary to increased levels of ionisation | T |
| The placenta is capable of metabolising some drugs presented to it | T |
| For highly lipid soluble drugs, degree of protein binding is an important factor in the rate of placental drug transfer | T |

| The EEG during sevoflurane anaesthesia has less “randomness” than when awake. | T |
| --- | --- |
| Propofol causes burst-suppression of the EEG at levels which have little effect on spinal reflexes. | T |
| When burst-suppression is induced by propofol, total brain oxygen consumption is reduced by up to 90%. | F 60% |
| Nitrous oxide causes similar changes to the EEG compared to sevoflurane. | F BIS values remain unchanged or even elevated during nitrous oxide anesthesia |
| Electrocortical silence cannot be produced with ketamine. | T |

T / F    The frontal EEG is a mixture of electrical signals derived from the cerebral cortex as well as sub-thalamic structures and the limbic system.

T / F     An “activated” EEG means that the amplitude of the EEG waveform is reduced.

T / F    The amplitude of an awake EEG is about the same as the p-wave on a standard ECG.

T / F    The amplitude of the EEG decreases with age.

T / F   “Burst suppression” is defined as periods of electrical activity alternating with periods of isoelectric EEG.

Pharmacodynamics

Define and explain dose-effect relationships of drugs with reference to:

· Graded and quantal response

· Therapeutic index

· Potency and efficacy

· Competitive and non-competitive antagonists

· Partial agonists, mixed agonist-antagonists and inverse agonists

· Additive and synergistic effects of drug combinations

BT_GS 1.4 Describe efficacy and potency with reference to dose- response curves

Use the curve below as a basis for your graphs

1.Add axes to indicate that the curve is a graded dose-response curve for a full agonist. Show the ED50.

2.Draw on the same axes, the same agonist in the presence of a competitive antagonist. Indicate the important features on your curve.

3.Show a partial agonist (E=0.8) which is equipotent to the full agonist drawn

4.Show on the graph with the full agonist, a more potent partial agonist capable of producing 50% maximal effect

5.Are any of the curves 2-4 compatible with a graph representing the full agonist in the presence of a non competitive (irreversible) antagonist? If so, which one?

| Fluoxetine significantly inhibits cytochrome P450 enzymes | T |
| --- | --- |
| Nortriptyline is usually better tolerated in the elderly than amitriptyline | F |
| There is a significant risk of serotinergic syndrome when SSRIs are given with tapentadol | F |
| Chronic lithium therapy has no effect on MAC of inhaled anaesthetics | F |
| Haemodyalisis is effective in the treatment of tricyclic antidepressant toxicity | F |

Pharmacokinetics

BT_PM 1.18 Describe the pharmacology of opioids deposited in the epidural space or cerebrospinal fluid

BT_RA 1.16 Describe the drugs which may be injected into the intrathecal or epidural space as adjuvant agents to a central neuraxial block and discuss their risks and benefits

| The analgaesic effect from continuous epidural infusion of hydroPHILIC opioids is primarily from systemic absorption | F |
| --- | --- |
| Intrathecal morphine provides analgaesia to more spinal levels than intrathecal fentanyl | T |
| Significant amounts of epidural morphine are sequestered in epidural fat | F relatively lipid insoluble |
| Epidural infusion of fentanyl may lead to systemic concentrations high enough to produce pharmacological effects | T |
| The peak period for respiratory depression with intrathecal morphine is from 18-24 hours after injection | T |
| Inaccurate drug delivery from the infusion pump contributes to 55% of the overall inaccuracy of a TCI infusion | T |
| With most modern TCI algorithms actual plasma concentrations are within 20-30% of predicted concentrations 95% of the time | T |
| The Marsh model uses age and weight to calculate the compartment size | T – age entered but not used in algorithm |
| The Schnider model may calculate a negative lean body mass in very obese patients | F |
| The most clinically reliable method is to target the effect site concentration observed at loss of consciousness. | T |

Respiratory

BT_PO 1.20

| A vital capacity breath is from expiratory reserve volume (ERV) to total lung capacity (TLC) | TRUE |
| --- | --- |
| The effort independent component is due to dynamic airways closure | T |
| The TLC is increased in obstructive lung disease | T |
| The effort independent component is steeper in restrictive lung disease | T |
| A normal peak expiratory flow rate would be 10 L/min | F |

Theme for the day: mindfulness.

| Deflation and Inflation of alveoli can be considered an exponential process | T |
| --- | --- |
| Fast” and “Slow” alveoli refers to the compliance/resistance relationship between the alveoli and its supplying bronchiole. | T |
| A relatively non compliant alveolus paired with a patent wide bronchiole will comprise a slow lung unit | Time constant = CR false |
| In the normal lung, alveoli in the apex are “slower” because compliance is less in the apex. | T |
| At fast respiratory rates, “slow” lung units may not have completed filling before the onset of expiration. | T |

| Work of breathing is a flow/volume relationship | T |
| --- | --- |
| The two main sources of impedance come from elastin and surface tension | T |
| Elastic work is greatest at a slow resp. frequency | F |
| Resistance work is greatest at a high resp frequency | T |
| Most energy consumed is dissipated as heat | 3% BMR 10% efficient |

BT_PO 1.93 Describe the physiology of sleep

| Arterial CO2 and O2 levels are unaffected by sleep | 3mmhg lower |
| --- | --- |
| Tidal volume reduces during sleep | T |
| General anaesthesia often disrupts sleep architecture on the first post-operative night | T – eliminate REM, increased phase 2 |
| Responsiveness to increased arterial CO2 is reduced by sleep | T |
| Loss of REM sleep on one night is often associated with increased REM sleep on subsequent nights | I HAVE NO IDEA. N = 1 . false |

BT_PO 1.11 Define compliance (static, dynamic and specific) and relate this to the elastic properties of the lung

.entry-header

Twice. So good, they named it twice.

| Most studies of lung compliance under anaesthesia indicate a decrease compared to the awake state | T |
| --- | --- |
| Age increases lung compliance | T |
| Dynamic compliance may be greater than static compliance | F |
| An elephant has greater compliance than a mouse | T – dynamic |
| Stress relaxation is a component of dynamic compliance | T |

| Minute ventilation is proportional to oxygen consumption at all levels of exercise | F |
| --- | --- |
| Response to hypoxaemia and hypercapnia are usually unaffected by obesity | T |
| FRC is reduced to a greater extent during anaesthesia, when a muscle relaxant is used than when one is not used | T |
| 1 MAC of anaesthesia preserves diaphragmatic function but can abolish EMG activity of other inspiratory muscles (If this is true how would this affect your anaesthesia plan for renal and ureteral lithotripsy?) | T – regional blood flow differences |
| Periodic breathing while asleep leading to oxygen saturations of 50% is common when first ascending over 4000 m | T |

SS_PA 1.24 Describe the physiology of the cardiovascular, respiratory, renal and neurological systems in the neonate and the changes that occur with growth and development and the implications of this for anaesthetic care

| During the first 2 weeks of age a neonate can flip back into a foetal circulation | T |
| --- | --- |
| The neonate has more compliant ventricles than an adult | F |
| Infants have more type I muscle fibres in their diaphragm | f |
| Neonates have decreased intracardiac calcium stores | T |
| Oxygen consumption in infants is higher than in adults | T |

Renal

Describe how the large daily volume of glomerular filtrate is altered by the kidney to form a relatively low volume of concentrated urine.

BT_PO 1.64    BT_PO 1.67

This question just asks for some basic functions of the kidney

| Approximately 80% of the glomerular filtrate is absorbed | F |
| --- | --- |
| The medulla is hyperosmotic due to salt and urea | T |
| ADH plays an important role | T |
| Urea is actively secreted | F – freely filtered But also secreted, so true? |
| The counter-current mechanism creates a hypo-osmotic medulla | F |

BT_RA1.12  Outline the factors determining intracranial pressure and discuss its regulation

| The Monroe- Kellie doctrine can be represented graphically as an elastance curve | T change in P/ change in vol |
| --- | --- |
| Reduction CSF production as ICP rises helps maintain a normal ICP | T |
| An intact blood brain barrier is necessary for intravenous mannitol to decrease brain water | T |
| Factors the reduce CMRO2 generally reduce cerebral blood volume | T |
| Doses of volatile anaesthetic agents less than 1MAC cause an uncoupling of the relationship between CMRO2 and cerebral blood flow | > 1 mac |

Outline the physiological effects of the adrenal hormones aldosterone and cortisol. (Do NOT describe synthesis or metabolism).

BT_PO 1.87

Sometimes we administer steroids… Sometimes we withhold them…

| Aldosterone works on the distal tubules and collecting ducts | T |
| --- | --- |
| Aldosterone increases the reabsorption of potassium | F |
| Cortisol increases blood glucose | t |
| Cortisol has mineralocorticoid activity | t |
| Both have slow onsets of action | t |

Outline the reference ranges for physiological and biochemical variables in pregnancy

| Uterine blood flow is approximately 20% of maternal cardiac output at term gestation | T |
| --- | --- |
| Maternal hyperventilation facilitates removal of CO2 from the foetus | T |
| Antithrombin III and factor XIII are decreased in pregnancy | 50% Factor 13 AT3 no change |
| Haemoglobin concentration and total red cell mass are decreased 15-20% in pregnancy | Hb conc decrease – dilutional anaemia |
| The word “glidescope” is pronounced the same as “kaleidoscope” | YES |

# Labour physiology and pharmacology

SB_OB 1.9 and SB_OB 1.8

Describe the influence of pregnancy on the pharmacokinetics and pharmacodynamics of drugs commonly used in anaesthesia and analgesia

Describe the anatomy and physiology of pain in labour and childbirth

Paracetamol is a category A drug in pregnancy, but care should still be taken in patients with pre-eclampsia. TRUE/FALSE

TRUE – “Paracetamol is a Category A medicine and is regarded as the analgesic of choice during pregnancy … However, it has been suggested that its potential influence on prostaglandin synthesis may have adverse effects in women at high risk of pre-eclampsia … A large Danish cohort study (n=98,140) suggested an increased risk of preterm birth following paracetamol exposure in early pregnancy in mothers with pre-eclampsia (OR = 1.55; 95%CI 1.16 to 2.07) but not in women without pre-eclampsia (OR 1.08; 95%CI 0.97 to 1.20)” (Acute Pain Management: Scientific Evidence)

The addition of lipid-soluble opioids to lumbar epidurals allows a reduction of local anaesthetic concentration. TRUE/FALSE

TRUE – “In labor epidurals the effects of morphine are not beneficial, and lipid soluble opioids such as fentanyl or sufentanyl are used instead. The effect of adding of opioids to local anesthetic mixture used for labor epidural if two fold. First, opioids improve the quality of pain relief by local anesthetics. Second, they allow to reduce the concentration of local anesthetics. This reduces undesirable side-effects, such as drop in blood pressure and muscle weakness that potentially can interfere with the second stage of labor.” (http://www.allaboutepidural.com/main-content/epidural-opioids)

Visceral pain receptors located in the lower uterine segment and cervix extend to the spinal segments of S2-S4. TRUE/FALSE

FALSE – cervix is S2-4 but uterus is around T11/12

Oxygen consumption increases by 40% during the first stage of labour. TRUE/FALSE

TRUE – exact figures vary between sources though

“Oxygen consumption increases above the pre-labor value by 40% in the first stage and by 75% in the second stage, secondary to the increased metabolic demands of hyperventilation, uterine activity, and maternal expulsive efforts.” (Chestnut's Obstetric Anesthesia: Principles and Practice)

Entonox side effects include maternal sedation. TRUE/FALSE.

TRUE

# Wednesday is always a good day for primary revision.

SS_OB 1.4 … again.

Describe the utero-placental circulation and the principles of placental physiology as related to placental gas exchange and regulation of placental blood flow.

The primary limitation to carbon dioxide transfer across the placenta is blood flow. TRUE/FALSE

TRUE – like CO2 in the lungs being ventilation-limited due to its high solubility, in the placenta it’s same but maternal blood flow is ‘ventilation’ for the fetus

That is, unless you consider the partial pressure gradient to be a separate factor (which it isn’t really, since blood flow is what maintains the partial pressure gradient)

Maternal-fetal exchange of most drugs and other substances occurs primarily by diffusion TRUE/FALSE

TRUE – “Most anesthetic drugs are passively transferred, with the rate of blood flow (hence drug delivery) affecting the amount of drug that crosses the placenta.” (Chestnut's Obstetric Anesthesia: Principles and Practice) – other sources support this for non-anaesthetic drugs also

Foetal O 2 saturation does not exceed 40% even with 100% O 2 delivery to the mother. TRUE/FALSE

FALSE – I don’t know if this is a trick question but the fetus has >40% O2 saturation without giving the mother 100% O2

The placenta synthesises progesterone from cholesterol. TRUE/FALSE

TRUE

O 2 delivery to the fetus is facilitated primarily because the fetal oxyhemoglobin dissociation curve is right-shifted. TRUE/FALSE.

FALSE – fetal OHDC is left-shifted compared with adult

# Obstetric Week

SS_OB 1.4 Describe the utero-placental circulation and the principles of placental physiology as related to placental gas exchange and regulation of placental blood flow.

Pregnant women have normal peripheral sympathetic nervous system activity. TRUE/FALSE

FALSE – “Pregnant women have greater sympathetic tone than nonpregnant women.” (Chestnut's Obstetric Anesthesia: Principles and Practice)

Uterine blood flow in the non pregnant state is 700mL/min. TRUE/FALSE

FALSE – “Uterine blood flow increases from a baseline value of approximately 50 mL/min to a level at term of 700 to 900 mL/min.” (Chestnut's Obstetric Anesthesia: Principles and Practice)

Increased maternal systemic resistance can lead to increased placental blood flow at term. TRUE/FALSE

FALSE? – I = V/R so if R increases then I would fall without a concomitant rise in V

Couldn’t find any direct answers but plenty that say increasing resistance reduces flow or that reducing resistance increases flow, e.g.

“Women who assume the supine position at term gestation experience a 10% to 20% decline in stroke volume and cardiac output”

“The decrease in blood viscosity from the lower hematocrit creates lower resistance to blood flow, which may be an essential component of maintaining the patency of the uteroplacental vascular bed.”

Or maybe the question is just wrong because maternal SVR normally falls.

A “normal” blood pressure reading in a supine (term) pregnant woman’s arm is a reliable indicator of uterine perfusion pressure. TRUE/FALSE

FALSE – potential aortocaval compression in supine position, which can impair uterine perfusion

“Women who assume the supine position at term gestation experience a 10% to 20% decline in stroke volume and cardiac output, consistent with the fall in right atrial filling pressure. Blood flow in the upper extremities is normal, whereas uterine blood flow decreases by 20% and lower extremity blood flow decreases by 50%. Perfusion of the uterus is less affected than that of the lower extremities because compression of the vena cava does not obstruct venous outflow via the ovarian veins” (Chestnut's Obstetric Anesthesia: Principles and Practice)

Additionally:

“Some term pregnant women exhibit an increase in brachial artery blood pressure when they assume the supine position, which is caused by higher systemic vascular resistance from compression of the aorta.” (Chestnut's Obstetric Anesthesia: Principles and Practice)

If there is no maternal hypotension then neuraxial blockade does not alter uterine blood flow. TRUE/FALSE

TRUE – “epidural anesthesia uncomplicated by arterial hypotension is associated with no alterations in uterine blood flow” (Shnider and Levinson's Anesthesia for Obstetrics p.31)

However, hypotension is more likely in pregnant patients: “Pregnancy increases dependence on the sympathetic nervous system for the maintenance of venous return and systemic vascular resistance. This, together with the effects of aortocaval compression, means that pregnant patients are particularly prone to hypotension and hemodynamic instability from sympathetic block induced by neuraxial anesthesia.” (Chestnut's Obstetric Anesthesia: Principles and Practice)

But sitting them to put it in (assuming it doesn’t take an hour) is OK: “Short intervals in the sitting position, such as occurs during epidural catheter placement, have no impact on uteroplacental blood flow.” (Chestnut's Obstetric Anesthesia: Principles and Practice)

# Obstetrics and the primary exam

SS_OB 1.1 Describe the physiological changes and their implications for anaesthesia that occur during pregnancy, labour and delivery, in particular the respiratory, cardiovascular, haematological and gastrointestinal changes.

Normal physiological changes begin in the first trimester of pregnancy. TRUE/FALSE

TRUE

The largest increase in cardiac output in a pregnant woman occurs immediately after delivery. TRUE/FALSE

TRUE

The closing capacity in normal pregnancy does not change. TRUE/FALSE

TRUE – “Whereas the closing capacity does not change during pregnancy, the FRC falls below the closing capacity in the supine position, causing areas of perfusion but no ventilation (intrapulmonary shunting), which also pre-disposes the mother to hypoxia.” (Anesthesia for Fetal Intervention and Surgery p. 132)

Gastrin is secreted by the placenta TRUE/FALSE

TRUE – “Gastrin production progressively increases during pregnancy, as it is produced by the placenta. Gastrin stimulates the secretion of water and enzymes from the gastrointestinal tract.” (Power and Kam p. 415)

Progesterone from the gestational sac may cause changes in the renin-angiotensin-aldosterone system in the first trimester, promoting sodium absorption and water retention. TRUE/FALSE

FALSE – “aldosterone secretion increases because of the natriuretic effect of progesterone” (Power and Kam p. 410)

# BT_PO 1.93 Describe the physiology of sleep

BT_PO 1.93 Describe the physiology of sleep

All quotes from CEACCP article ‘Physiology of Sleep’

Arterial CO2 and O2 levels are unaffected by sleep TRUE/FALSE

FALSE – “During NREM sleep, there is a decrease in respiratory drive and a reduction in the muscle tone of the upper airway leading to a 25% decrease in minute volume and alveolar ventilation and a doubling of airway resistance accompanied by a small (0.5 kPa) increase in PaCO2 and decrease in PaO2. Hypercarbic and hypoxic ventilatory drives are reduced compared with wakefulness.”

Tidal volume reduces during sleep TRUE/FALSE

TRUE – though it can be variable

General anaesthesia often disrupts sleep architecture on the first post-operative night TRUE/FALSE

TRUE – though it may not actually be due to GA: “Anaesthesia and surgery can have a profound effect upon sleep. On the first night after surgery, sleep architecture is severely disrupted with little or no SWS and REM sleep. The light Stage 2 sleep is fragmented with frequent awakenings. The degree of disruption appears to be related to the severity of the surgical insult. The mechanism is unclear but it is probably due to a combination of the surgical stress and the effects of opioid analgesics.”

Responsiveness to increased arterial CO2 is reduced by sleep TRUE/FALSE

TRUE – see quote above

Loss of REM sleep on one night is often associated with increased REM sleep on subsequent nights TRUE/FALSE

TRUE – “Recovery of lost SWS and REM sleep occurs on postoperative nights 2–5”

# BT_RA 1.12 Determinants of ICP and their regulation

BT_RA1.12  Outline the factors determining intracranial pressure and discuss its regulation

The Monroe- Kellie doctrine can be represented graphically as an elastance curve TRUE/FALSE

Reduction CSF production as ICP rises helps maintain a normal ICP TRUE/FALSE

FALSE – see post further below with graph

An intact blood brain barrier is necessary for intravenous mannitol to decrease brain water TRUE/FALSE

TRUE

Factors the reduce CMRO2 generally reduce cerebral blood volume TRUE/FALSE

TRUE – provided autoregulation is intact

Doses of volatile anaesthetic agents less than 1MAC cause an uncoupling of the relationship between CMRO2 and cerebral blood flow TRUE/FALSE

FALSE – though I don’t like the generality of the question. Halothane does, so perhaps it’s true? Isoflurane and sevo don’t. Des probably doesn’t either (it transitions somewhere between 0.5 MAC and 1.5 MAC so it could)

“While volatile anaesthetic agents are intrinsic vasodilators, they also decrease CMRO2 in a dose-dependent manner. Therefore, in the presence of intact flow-metabolism coupling, volatiles cause a coupled decrease in both CMRO2 and CBF. The decrease in CBF caused by coupling is opposed by the vasodilatory effect of these agents, ultimately resulting in either no change or small decrease in CBF at low minimum alveolar concentration (MAC). However, CBF increases with MAC after metabolic suppression is maximal. Volatile agents were previously believed to uncouple flow-metabolism coupling but in fact the CBF/CMR ratio is altered or, more strictly, increased. At the same MAC dose, CMRO2 decreases much more than the decrease in CBF

At 0.5 MAC, isoflurane, desflurane, and sevoflurane minimally delay, but preserve the cerebral autoregulation, whereas at 1.5 MAC autoregulation is considerably reduced by isoflurane and desflurane. Sevoflurane, in contrast, produces much lesser cerebral vasodilation and delays but preserves the autoregulatory response even at 1.5 MAC, making it the favoured volatile agent during neuroanaesthesia.” (CEACCP ‘Cerebral Physiology’)

# 2017.1 : SAQ 12

Discuss the physiological consequences of total spinal anaesthesia caused by intrathecal administration of 20ml of  2% lignocaine at the L3/4 level. (Do not include management)

BT_RA 1.2

Mainly from the referenced article about total spinal anaesthesia we discussed previously

This will cause bradycardia TRUE/FALSE

TRUE

The patient will have dilated gut TRUE/FALSE

FALSE

The patient will become hyperthermic TRUE/FALSE

FALSE

The patient will have dilated pupils TRUE/FALSE

TRUE

The patient will be unconscious TRUE/FALSE

TRUE

# SS_OB 1.2

Outline the reference ranges for physiological and biochemical variables in pregnancy

Uterine blood flow is approximately 20% of maternal cardiac output at term gestation TRUE/FALSE

FALSE – as above, at term uterine blood flow is 700-900 mL/min and CO is 50% above baseline. Can’t be 20% (unless one of those is wrong)

Maternal hyperventilation facilitates removal of CO2 from the foetus TRUE/FALSE

TRUE – maintains concentration gradient

Antithrombin III and factor XIII are decreased in pregnancy TRUE/FALSE

TRUE – part one course lecture

Haemoglobin concentration and total red cell mass are decreased 15-20% in pregnancy TRUE/FALSE

FALSE – red cell mass increases

The word “glidescope” is pronounced the same as “kaleidoscope” TRUE/FALSE

I think this could catch on.

BT_PM1.18 : neuraxial opioids

BT_PM 1.18 Describe the pharmacology of opioids deposited in the epidural space or cerebrospinal fluid

With regard to epidural fentanyl (1 mcg.kg-1) plasma concentrations of fentanyl are similar to the same dose given intramuscularly TRUE/FALSE

TRUE – “epidural fentanyl absorption into the systemic circulation is biphasic, producing plasma arterial fentanyl concentrations similar to those observed after intramuscular drug administration” (Evers & Maze)

the duration of analgesia is 1 hour TRUE/FALSE

FALSE – “ its duration of action is 2-5 hours.”

the CSF concentrations of fentanyl are maximal at 50 minutes  TRUE/FALSE

the incidence of pruritis is greater than following an equi-analgesic dose of epidural morphine  TRUE/FALSE

FALSE – pruritis is more common with IT morphine (one source says 60% incidence)

the onset of analgesia is around 2-5 minutes because fentanyl is highly lipid soluble  TRUE/FALSE

FALSE – “It has a rapid onset of action 15-30 minutes”

Intrathecal fentanyl has fewer adverse effects than intrathecal morphine TRUE/FALSE

FALSE – morphine has more

Intrathecal morphine produces better postoperative analgesia than intrathecal fentanyl after LSCS   TRUE/FALSE

TRUE – couldn’t find a textbook answer but there are various (flawed) journal articles indicating this, e.g.

http://www.obstetanesthesia.com/article/S0959-289X(97)80051-9/pdf

http://www.e-mjm.org/2009/v64n1/Intrathecal_Morphine.pdf

Doses of intrathecal morphine greater than 50mcg have no greater benefits in terms of analgesia but carry a higher incidence of respiratory depression    TRUE/FALSE

FALSE – standard doses are > 50mcg

100 mcg of morphine is 0.1ml of morphine 10mg/ml    TRUE/FALSE

FALSE – 10mg/mL = 1mg/0.1mL

extended-release epidural morphine is associated with less respiratory depression than IVPCA morphine    TRUE/FALSE

FALSE – “Although perioperative single-dose epidural EREM (versus IV-PCA) was effective for postoperative pain relief for up to 48 hours, it is associated with significantly higher odds of respiratory depression. Further examination of the issue of respiratory depression of epidural EREM may be warranted.” (https://www.ncbi.nlm.nih.gov/pubmed/19947071)

SS_PA 1.24 : paediatric physiology

Not a bad textbook, Evers & Maze… But rubbish for paediatric pharmacodynamics… So I’ve swapped over to some paediatric physiology instead. I used Miller as it was handy.

All answers from Miller

SS_PA 1.24 Describe the physiology of the cardiovascular, respiratory, renal and neurological systems in the neonate and the changes that occur with growth and development and the implications of this for anaesthetic care

During the first 2 weeks of age a neonate can flip back into a foetal circulation   TRUE/FALSE

FALSE – it’s not a fetal circulation without a placenta!

Having said that, the answer they want is probably ‘true’ given that the very first sentence in Miller’s chapter on paeds is: “During the first few weeks of life, neonates are vulnerable to a phenomenon, the flip-flop circulation; that is, going from an adult type of circulation to a fetal type.”

The neonate has more compliant ventricles than an adult   TRUE/FALSE

FALSE – “The reduced cellular mass of the neonatal heart devoted to contractility results in less compliant ventricles.”

Infants have more type I muscle fibres in their diaphragm   TRUE/FALSE

FALSE – “Another important factor is the composition of the diaphragmatic and intercostal muscles. These muscles do not achieve the adult configuration of type I muscle fibers until the child is approximately 2 years old.  Because type I muscle fibers provide the ability to perform repeated exercise, any factor that increases the work of breathing contributes to early fatigue of the respiratory muscles of infants; this partially explains why the infant’s respiratory rate and hemoglobin desaturation is so rapid, and their propensity to develop fatigue and apnea with airway obstruction.”

Neonates have decreased intracardiac calcium stores   TRUE/FALSE

TRUE – “cardiac calcium stores are reduced because of the immaturity of the sarcoplasmic reticulum; consequently, neonates have a greater dependence on exogenous (blood-ionized) calcium and probably increased susceptibility to myocardial depression by volatile anesthetics that have calcium channel–blocking activity”

Oxygen consumption in infants is higher than in adults   TRUE/FALSE

FALSE – Miller says infants’ “oxygen consumption is two to three times higher” than adults – but surely that is only if indexed to some measure of their size? No way do I use less oxygen than an infant. Anyway, again, if it’s based on Miller the answer is probably ‘true’.

SS_PA 1.51 : paediatric pharmacokinetics

SS_PA 1.51  : Describe how the pharmacokinetics of drugs commonly used in anaesthesia in neonates and children differ from adults and the implications for anaesthesia

All answer from Evers & Maze

Oral medications are absorbed slowly in infants due to decreased gastric emptying and intestinal motility  TRUE/FALSE

TRUE – “The rate at which most drugs are absorbed when given by the oral route is slower in neonates and young infants than in older children because gastric emptying is delayed”

Nasal midazolam tastes good  TRUE/FALSE

Surely it’s a matter of opinion… “Nasal midazolam, for example, has a bitter taste”

Gastric pH is lower in infants than in adults  TRUE/FALSE

FALSE – “Gastric pH is elevated (> 4) in neonates, increasing the bioavailability of acid-labile compounds (e.g., penicillin G) and decreasing the bioavailability of weak acids (e.g., phenobarbital) when given orally”

The solubility coefficients of an inhaled anaesthetic will determine its volume of distribution  TRUE/FALSE

TRUE – “Because solubility determines the volume of distribution, it has considerable effect on the uptake of inhalation drugs in children”

In Evers & Maze the author states  ‘Other factors causing a more rapid “wash-in” of inhalational anesthetics include the greater fraction of cardiac output distributed to the vessel-rich tissue group (e.g., the lungs)’ What do you think of this comment?

(hint – keep reading this chapter on paediatric pharmacology – you might even be able to spend the rest of the week testing your retention of the material)

BT_PM 1.15 : Routes of opioid administration

BT_PM 1.15  Discuss the pharmacokinetic and clinical implications of different routes of administration for commonly used opioids, including the oral, transdermal, subcutaneous, intramuscular and intravenous routes, and with particular reference to fentanyl, morphine, methadone, tramadol and codeine

Fentanyl undergoes significant first pass pulmonary uptake and metabolism.  TRUE/FALSE

FALSE? – “Extensive pulmonary uptake of lipophilic basic amines, such as fentanyl, attenuates early blood drug concentrations after rapid intravenous administration” but I don’t think it undergoes significant metabolism there

The cytochrome P450 3A4 (CYP3A4) is predominantly responsible for the metabolism of Alfentanil.   TRUE/FALSE

TRUE

Alfentanil undergoes extensive hepatic metabolism that demonstrates extensive interindividual variability   TRUE/FALSE

FALSE – CYP3A4 has minimal clinical variability

The bioavailability of sublingual buprenorphine is similar to that of parenteral buprenorphine   TRUE/FALSE

FALSE – around 30-40% (depending on source)

Epidural fentanyl undergoes a biphasic absorption pattern   TRUE/FALSE

TRUE – “epidural fentanyl absorption into the systemic circulation is biphasic, producing plasma arterial fentanyl concentrations similar to those observed after intramuscular drug administration” (Evers & Maze)

SS_OB 1.13 Placental transfer of drugs

SS_OB 1.13 Explain the factors that influence the transfer of drugs across the placenta

Ionised drugs are more likely to cross the placenta compared with non-ionised drugs T/F

TRUE

Heparin is safe to use in pregnancy because its large molecular size prevents it crossing the placenta T/F

TRUE – Miller says “Both heparin and glycopyrrolate have minimal placental transfer because they are highly charged” but I’m pretty sure the size is also a factor

Weakly basic drugs, with a pKa less than 7.4, may become concentrated in the fetal compartment secondary to increased levels of ionisation T/F

TRUE? – It can definitely occur with weakly basic drugs with pKa > 7.4 (e.g. local anaesthetics). Weakly basic drugs with pKa < 7.4 would be mostly unionised in the maternal circulation but I don’t see why they wouldn’t still concentrate in the fetus if its pH were significantly lower, unless the pKa was much lower than 7.4 (maybe < 6.4)

The placenta is capable of metabolising some drugs presented to it T/F

TRUE – “Tthe extent to which drugs cross the placenta is also modulated by the actions of placental phase I and II drug-metabolising enzymes, which are present at levels that fluctuate throughout gestation.” (https://www.ncbi.nlm.nih.gov/pubmed/15170365)

For highly lipid soluble drugs, degree of protein binding is an important factor in the rate of placental drug transfer T/F

TRUE – “Highly protein-bound drugs are affected by the concentration of maternal and fetal plasma proteins” (Chestnut's Obstetric Anesthesia: Principles and Practice)

BT_GS 1.51 Describe the concept of depth of anaesthesia and how this may be monitored

BT_GS 1.51 Describe the concept of depth of anaesthesia and how this

may be monitored

BT_GS 1.52 Explain the principles involved in the electronic monitoring of

depth of sedation and anaesthesia, including the use of EEG

analysis

What is “depth of anaesthesia”? How is it different from “level of consciousness”?

Can you define these concepts in a way that does not rely on an electronic device?

T / F	Depth of anaesthesia refers to the effect of anaesthetic drugs on the brain only.

I can’t find a definite answer to this as sources disagree on what ‘depth of anaesthesia’ means and even if such a concept exists at all. Could be either true or false depending on where you read.

T / F	The BIS index is generated by combining together at least 3 different measures of EEG activity.

TRUE – “BIS combines information from three EEG analyses: the spectrogram, the bispectrum, and a time domain assessment of burst suppression” (Miller)

T / F	A BIS index under 60 means that a patient will not respond to voice.

FALSE – it should indicate that but it’s an imperfect monitor. From Miller: “In the case of dexmedetomidine, slow oscillations are prominent during sedation, with BIS values that are typically in the unconscious range. However, the patient can be readily aroused by verbal commands or light shaking because dexmedetomidine does not produce profound unconsciousness.”

T / F	The BIS index decreases during natural sleep, but not below 50.

False – one study found that “Light sleep occurred at BIS values of 75-90, slow-wave sleep occurred at BIS values of 20-70, and rapid eye movement sleep occurred at BIS values of 75-92. The effects of natural sleep on the BIS seem to be similar to the effects of general anesthesia on the BIS. The BIS may have a role in monitoring depth of sleep.”

T / F	Both the BIS and Entropy monitors analyse EEGs using frequencies at which muscle activity is significant.

TRUE – though both have methods of (potentially) differentiating EMG activity

T / F   The Spectral Edge Frequency is unaffected by muscle relaxation.

TRUE – “Neuromuscular blockade by atracurium bolus injection, after LOC, induced decreases in BIS, EMG, and RE but not SEF or SE.” https://www.ncbi.nlm.nih.gov/pubmed/16301247

BT_GS 1.52 Explain the principles involved in the electronic monitoring of depth of sedation, including EEG analysis.

T / F	The EEG during sevoflurane anaesthesia has less “randomness” than when awake.

TRUE – “A readily apparent feature of the EEG, as patients proceed through deeper levels of unconsciousness induced by general anesthesia, is that the patterns become more regular and ordered. That is, we observe an apparent decrease in the entropy of the EEG signal.” (Miller)

T / F	Propofol causes burst-suppression of the EEG at levels which have little effect on spinal reflexes.

I feel like the answer is somewhere in this text but it’s hard to find… I’m inclined to say false. https://tampub.uta.fi/bitstream/handle/10024/67372/951-44-5951-2.pdf?sequence=1

T / F	When burst-suppression is induced by propofol, total brain oxygen consumption is reduced by up to 90%.

FALSE – “escalating propofol doses lead to burst suppression on the EEG with a decrease of CBF by 38-58% and CMRO2 by 22-43%”

T / F	Nitrous oxide causes similar changes to the EEG compared to sevoflurane.

FALSE – Miller describes them separately and doesn’t seem to directly compare but the effects sound quite different and it would make sense given they are pharmacodynamically quite different. Nitrous also doesn’t affect BIS in the same way so I’d be surprised if overall EEG effects were ‘similar’: “Nitrous oxide increases the amplitude of high-frequency EEG activity 26 and decreases the amplitude of low-frequency EEG activity, yet it has little to no effect on the BIS index”

T / F	Electrocortical silence cannot be produced with ketamine.

TRUE – “Electrocortical silence cannot be produced with ketamine.”

BT_PO 1.94 Outline the basis of the electroencephalogram

Useful resources include the 8th Ed of Miller, especially Chapter 17, which is available online from the ANZCA website.

T / F    The frontal EEG is a mixture of electrical signals derived from the cerebral cortex as well as sub-thalamic structures and the limbic system.

Not sure. I can’t find a clear answer to this but the limbic system and sub-thalamic structures seem pretty deep to be generating measurable potentials over the frontal skin.

T / F     An “activated” EEG means that the amplitude of the EEG waveform is reduced.

FALSE – “When events occur that lead the brain to produce higher frequencies and larger amplitudes, the EEG is described as activated, and when slower frequencies are produced (theta = 4 to 7 Hz, and delta = <4 Hz), the EEG is said to be depressed.”

T / F    The amplitude of an awake EEG is about the same as the p-wave on a standard ECG.

TRUE – “Amplitude is the size, or voltage, of the recorded signal and ranges commonly from 5 to 500 μV (versus 1 to 2 mV for the electrocardiogram signal)” – given the p wave is much smaller than the R/S waves it looks about right

T / F    The amplitude of the EEG decreases with age.

TRUE – “Because neurons are irreversibly lost during the normal aging process, EEG amplitude decreases with age.”

T / F   “Burst suppression” is defined as periods of electrical activity alternating with periods of isoelectric EEG.

TRUE

SS_OB 1.9 Drug Variability in Pregnancy

SS_OB 1.9 Describe the influence of pregnancy on the pharmacokinetics and pharmacodynamics of drugs commonly used in anaesthesia and analgesia

TRUE/FALSE In pregnancy the average gain of 8 litres of total body water significantly increases the volume of distribution of hydrophilic drugs.

TRUE – I spent far too long trying to confirm the average gain of 8L TBW but there are enough sources in that ballpark to indicate it’s probably true

TRUE/FALSE Foetal and placental tissues provide another compartment for drug distribution.

TRUE

TRUE/FALSE Pseudocholinesterase activity is decreased in pregnancy causing prolongation of succinylcholine block.

TRUE

TRUE/FALSE Pregnancy reduces MAC by 25-30%.

TRUE – thereabouts, though may be up to 40%

TRUE/FALSE Nociceptive response thresholds are elevated in pregnancy.

TRUE – “Pregnancy is associated with increases in nociceptive response thresholds that are mediated by endogenous opioid systems.”

https://books.google.com.au/books?id=FMU0AwAAQBAJ&pg=PA307&lpg=PA307&dq=Nociceptive+response+thresholds+pregnancy&source=bl&ots=cx_eRZDpjj&sig=LDMWi9ayaPzIya4AW-3B2JLPM2U&hl=en&sa=X&ved=0ahUKEwj1zrqPgLjUAhVErJQKHe-FAVUQ6AEINzAC#v=onepage&q&f=false

BT_PO 1.97 Describe the dynamics and metabolism of cerebrospinal fluid

BT_PO 1.97 Describe the dynamics and metabolism of cerebrospinal fluid

Refer to the diagram below, which shows CSF formation and absorption to help answer the questions

The blue line represents CSF production, which is independent of intraventricular pressure T/F

TRUE – blue line is production and red line is resorption. Ganong (which the LO post references) says:

“Lumbar CSF pressure is normally 70–180 mm H2O. Up to pressures well above this range, the rate of CSF formation is independent of intraventricular pressure.” – the blue line being flat indicates this

CSF reabsorption is zero when CSF pressure is low T/F

TRUE – from Ganong:

“absorption is proportional to the pressure. At a pressure of 112 mm H2O, which is the average normal CSF pressure, filtration and absorption are equal. Below a pressure of approximately 68 mm H2O, absorption stops.” – the red line plateauing to zero indicates this

At normal CSF presssure, production of CSF is greater than reabsorption T/F

TRUE – see range between dotted lines on graph

Increased absorption of CSF is a indefinite means of compensating for rising ICP T/F

FALSE – I don’t quite understand the question but there is a limit to how much CSF you can reabsorb and some CSF is required to continue functioning so it sounds false

CSF is a filtrate of plasma  T/F

FALSE – produced by choroid plexus from filtration of plasma through fenestrated capillaries then active transport of water and dissolved substances through epithelial cells of blood-CSF barrier

BT_PO 1.99 Some neuropharmacology

BT_PO 1.99   Outline the pharmacology of anti-depressant, anti- psychotic, anti-convulsant, anti-parkinsonian and anti- migraine medication

Fluoxetine significantly inhibits cytochrome P450 enzymes T/F

TRUE – CYP2D6 inhibitor

From the tramadol article I sent earlier:

“Inhibition of CYP2D6 enzymes by SSRIs prevents the hepatic metabolism of tramadol. This elevates the concentration of the parent compound and increases its serotonergic effects in the brain. SSRIs that are strong inhibitors of CYP2D6, such as sertraline paroxetine and fluoxetine increase the risk of serotonin syndrome when taken with tramadol.”

So giving tramadol is more likely to be problematic with SSRIs that inhibit CYP2D6, in addition to just the additive serotonergic effects.

Nortriptyline is usually better tolerated in the elderly than amitriptyline T/F

TRUE – “Adverse effects appear to be most common with amitriptyline, and so nortriptyline may be preferred in this patient group” (Acute Pain Management: Scientific Evidence)

There is a significant risk of serotinergic syndrome when SSRIs are given with tapentadol T/F

FALSE – tapentadol is noradrenergic but very minimally serotonergic

“From post-marketing data of >1.4 million patients, only 2 cases of serotonin syndrome (both in patients taking tapentadol ER concomitantly with 2 potentially serotonergic drugs) were identified”

Chronic lithium therapy has no effect on MAC of inhaled anaesthetics T/F

FALSE – not sure about acute/chronic but lithium decreases MAC

Haemodyalisis is effective in the treatment of tricyclic antidepressant toxicity T/F

FALSE – high PB, large Vd

BT_RA 1.16 Neuraxial Opiates

BT_PM 1.18 Describe the pharmacology of opioids deposited in the epidural space or cerebrospinal fluid

BT_RA 1.16 Describe the drugs which may be injected into the intrathecal or epidural space as adjuvant agents to a central neuraxial block and discuss their risks and benefits

Most of these are answered in this surprisingly good article:

http://www.paincommunitycentre.org/article/epidural-mode-action-local-anaesthetics-and-opioids-epidural

TRUE/FALSE The analgaesic effect from continuous epidural infusion of hydroPHILIC opioids is primarily from systemic absorption

FALSE – proven by greater effect from epidural administration compared with same dose systemically (despite lower plasma concentrations)

TRUE/FALSE Intrathecal morphine provides analgaesia to more spinal levels than intrathecal fentanyl

TRUE – remains in CSF for longer and at higher concentrations à greater spread

TRUE/FALSE Significant amounts of epidural morphine are sequestered in epidural fat

FALSE – not very lipophilic

TRUE/FALSE Epidural infusion of fentanyl may lead to systemic concentrations high enough to produce pharmacological effects

TRUE

TRUE/FALSE The peak period for respiratory depression with intrathecal morphine is from 18-24 hours after injection

FALSE – peak at 6h

https://academic.oup.com/bjaed/article/8/3/81/293391/Intrathecal-opioids-in-the-management-of-acute

BT_PO 1.51 Autonomic nervous system

BT_PO 1.51

Describe the autonomic nervous system and its physiological roles including:

· Autonomic receptors and cellular effects of receptor activation

· Autonomic transmitters, their synthesis, release and fate

TRUE/FALSE  Sympathetic stimulation increases peristalsis

FALSE – reduces peristalsis/motility

TRUE/FALSE  The nicotinic receptor sub-type found in autonomic ganglia is a G-protein coupled receptor

FALSE – pentameric ligand-gated cation channel

TRUE/FALSE  The acetylcholine muscarinic receptor is an example of a G-Protein coupled receptor

TRUE – M1,3,5 = Gq; M2,4 = Gi

TRUE/FALSE  Noradrenaline is the neurotransmitter released from all post-ganglionic sympathetic neurons

FALSE – acetylcholine at sweat glands

TRUE/FALSE  The sino-atrial node is innervated by the right vagus

TRUE – usually

BT_PO 1.51 Autonomic nervous system

BT_PO 1.51

Describe the autonomic nervous system and its physiological roles including:

· Autonomic receptors and cellular effects of receptor activation

· Autonomic transmitters, their synthesis, release and fate

TRUE/FALSE  Acetylcholine is the neurotransmitter released from all pre-ganglionic autonomic neurons

TRUE

TRUE/FALSE  Acetylcholine is released from all post-ganglionic parasympathetic neurons except those that innervate sweat glands

FALSE – it is released at sweat glands too

TRUE/FALSE  The adrenal medulla is innervated by inhibitory parasympathetic neurons

FALSE

TRUE/FALSE  Plasma concentration of acetylcholine are low predominantly due to pseudocholinesterase

FALSE – pseudocholinesterase doesn’t metabolise acetylcholine – it metabolises sux, mivacurium, procaine, cocaine

TRUE/FALSE  Nicotinic acetylcholine receptors mediate transmission at autonomic ganglia

TRUE

SS_PA 1.21 Describe the foetal circulation

Within the umbilical cord, there are two veins, and one artery. TRUE / FALSE

FALSE – 2 arteries, 1 vein

Foetal blood returning from the placenta has an oxygen saturation of 80%. TRUE / FALSE

TRUE – primary course lecture (neonatal physiology)

The foramen ovale remains open in the foetus under the influence of prostaglandins. TRUE / FALSE

FALSE – Foramen ovale functionally closes due to ↑left-sided pressures + ↓right-sided pressures. Ductus arteriosus closes due to ↓PG (esp. PGE2) and ↑pO2

The ductus arteriosus adds blood into the aorta distal to the vessels supplying the brain. TRUE / FALSE

TRUE

Following delivery, there is a “transitional” circulation. Target SpO2 levels for a neonate are: 70-90% at 3 mins, and 85-90% at 10 mins. TRUE / FALSE

Values are close enough to be true? Miller:

| Time after Birth (min) | Target SpO 2 (%) |
| --- | --- |
| 1 | 60-65 |
| 2 | 65-70 |
| 3 | 70-75 |
| 4 | 75-80 |
| 5 | 80-85 |
| 10 | 85-95 |

BT_PM 1.1 Anatomy of sensory pathways

BT_PM 1.1 Describe the anatomy of the sensory pathways with particular reference to pain sensation

The spinothalamic tracts are in the dorsal column TRUE/FALSE

FALSE – anterolateral

Primary afferent neurons synapse in the dorsal root ganglion TRUE/FALSE

FALSE – dorsal root ganglion contains cell bodies

C-fibres synapse in the substantia gelatinosa TRUE/FALSE

TRUE

Pain and temperature fibres decussate at the level of the medulla TRUE/FALSE

FALSE – pain and temperature = spinothalamic = decussate in spinal cord

C-fibres are unmyelinated TRUE/FALSE

TRUE

BT_PM 1.26 NMDA Receptors

BT_PM 1.26 Describe the location and role of NMDA receptors

Most of these were covered in the primary course lecture on pain

TRUE/FALSE The NMDA receptor is involved in development of tolerance to opioids.

TRUE (Stoelting p. 822)

TRUE/FALSE Glutamate acts at NMDA receptors in the dorsal horn.

TRUE

TRUE/FALSE The resting NMDA receptor is blocked by magnesium.

TRUE

TRUE/FALSE Activation of postsynaptic NMDA receptors causes influx of sodium and calcium.

TRUE

TRUE/FALSE Presynaptic NMDA receptor activation reduces cell excitability.

FALSE – not entirely clear but appears that presynaptic NMDARs “facilitate presynaptic neurotransmitter release”

BT_PM 1.8 Pain in the Elderly

BT_PM 1.8 Describe the alterations to physiology and perception of pain in the older patient

All questions are answered directly (quotes) in Acute Pain Management: Scientific Evidence

TRUE/FALSE Opioid receptor density is decreased in the brain and spinal cord of the elderly

FALSE – “Opioid-receptor density is decreased in the brain but not in the spinal cord, and there may be decreases in endogenous opioids.”

TRUE/FALSE Older people have a reduced ability to tolerate intense pain

TRUE – “Studies looking at age-related changes in pain tolerance are limited, but in general, using a variety of experimental pain stimuli, there is a reduced ability in older people to endure or tolerate intense pain”

TRUE/FALSE Autonomic responses to pain are blunted in dementia

TRUE – “autonomic responses typically associated with the onset of acute pain (ie increased heart rate, blood pressure, galvanic skin resistance, breathing) appear to be blunted in persons with dementia”

TRUE/FALSE There is a lower threshold for temporal summation of painful thermal stimuli in the elderly

TRUE – “Temporal summation of thermal stimuli was increased in the older compared with younger subjects.”

I think ‘Temporal summation of thermal stimuli was increased’ = ‘There is a lower threshold for temporal summation of painful thermal stimuli’ – anyone disagree?
Niki: I feel like it means the opposite, cause when old people get in the bath it takes them longer to realise they are getting burnt. The working of the sentence is ambiguous though, could mean that stimulation of the same amount is summated to more stimulation and therefore threshold lower, or could mean that you need more stimulation overall stimulus… Where was it from?

TRUE/FALSE Primary hyperalgaesia resolves more slowly in the elderly than the young

FALSE – “After topical application of capsaicin, the magnitude and duration of primary hyperalgesia was similar on both older and younger subjects but secondary hyperalgesia (tenderness) resolved more slowly in older people.”

SS_PA 1.1 Paediatric Airway Anatomy

SS_PA 1.1 Describe the anatomy of the neonatal airway, how this changes with growth and development and the implications for airway management

TRUE/FALSE Under extension of the neck may cause airway obstruction in the neonate

TRUE

TRUE/FALSE Over extension of the neck may cause airway obstruction in the neonate

TRUE

TRUE/FALSE Infants are obligate nasal breathers

TRUE

Miller p. 2762 “Although infants are obligate nasal breathers, approximately 8% of preterm neonates (31 to 32 weeks’ postconceptual age [PCA]) and 40% of term infants can convert to oral breathing if the nasal airway is obstructed. Almost all infants can easily convert to oral breathing by 5 months of age; most convert to oral breathing if the obstruction lasts longer than 15 seconds.”

I don’t really understand their use of terminology – doesn’t obligate mean that you have to do it and thus can’t do anything else? Like how an obligate aerobe needs oxygen?

TRUE/FALSE Neonates have a large tongue in comparison to the oropharynx

TRUE

TRUE/FALSE Infants are unable to breathe via the mouth

FALSE (as above)

Nerve Conduction

BT_RA 1.1  Describe the physiology of nerve conduction

Entry of Na+ into the nerve cell during an action potential, causes further Na+ to enter the cell in a positive feedback loop TRUE/FALSE

TRUE – that’s why depolarisation is so rapid after reaching threshold potential

A nerve cell membrane is charged at rest, with negative charges aligned along the outside of the nerve cell membrane TRUE/FALSE

FALSE – inside is negative, outside is positive

Propagation of an nerve action potential is unidirectional along the conducting nerve fibre TRUE/FALSE

TRUE – due to refractory period preventing retrograde transmission

Saltatory conduction refers to conduction of the action potential in unmyelinated nerve fibres TRUE/FALSE

FALSE – salutatory conduction is in myelinated fibres

During the absolute refractory period, a supra maximal stimulus is required to elicit an action potential TRUE/FALSE

FALSE – absolute refractory period is absolutely refractory… supramaximal stimulus may work during relative refractory period